MODEL 3

2024

OWNER'S MANUAL

Software version: 2023.44.30

Australia

YOUR OWNER'S MANUAL

For the latest and greatest information that is customized to your vehicle, view the Owner’s Manual on your vehicle’s  touchscreen by touching the app launcher and then selecting the Manual app. The information is specific to your vehicle  depending on the features you purchased, vehicle configuration, market region, and software version. In contrast, owner  information that is provided by Tesla elsewhere is updated as necessary and may not contain information unique to your  vehicle.

RELEASE NOTES

Information about new features is displayed on the touchscreen after a software update, and can be viewed at any time  by choosing the Release Notes tab in the Manual app, or by touching Controls > Software > Release Notes. If the  content in the Owner's Manual on how to use your vehicle conflicts with information in the Release Notes, the Release  Notes take precedence.

ILLUSTRATIONS AND PRODUCT SPECIFICATIONS

The illustrations provided in this document are for demonstration purposes only. Depending on vehicle options, software  version and market region, the information displayed on the touchscreen in your vehicle may appear slightly different.

All specifications and descriptions contained in this document are verified to be accurate at the time of printing.  However, because continuous improvement is a goal at Tesla, we reserve the right to make product modifications at any  time. To communicate any inaccuracies or omissions in this document, please send an email to:  ownersmanualfeedback@tesla.com.

SAFETY INFORMATION

You can find safety information in your Model 3 Owner's Manual on the touchscreen.

For detailed information about your Model 3, go to the Tesla website for your region, log on to your Tesla account, or  sign up to get an account.

If you have any questions or concerns about your Model 3, call Tesla. To find the number for your region, go to  www.tesla.com, then view contact information.

© 2012-2024 TESLA, INC.

All information in this document and all vehicle software is subject to copyright and other intellectual property rights of  Tesla, Inc. and its licensors. This material may not be modified, reproduced or copied, in whole or in part, without the  prior written permission of Tesla, Inc. and its licensors. Additional information is available upon request. Shown here are  trademarks or registered trademarks of Tesla, Inc. in the United States and other countries:

Overview................................................................ 3 Exterior................................................................................................3 Interior Overview.............................................................................4 Touchscreen.......................................................................................6 Interior Electronics......................................................................... 11 Car Status..........................................................................................13 Voice Commands...........................................................................16 Cameras.............................................................................................18

Opening and Closing....................................... 19 Keys.....................................................................................................19 Doors..................................................................................................22 Windows...........................................................................................25

Storage Areas.................................................... 26 Rear Trunk........................................................................................26 Front Trunk...................................................................................... 28 Interior Storage............................................................................. 30

Seating and Safety Restraints...................... 31 Front and Rear Seats....................................................................31 Seat Belts.........................................................................................34 Child Safety Seats.........................................................................37 Airbags..............................................................................................46

Connectivity.......................................................49 Mobile App......................................................................................49 Wi-Fi....................................................................................................51 Bluetooth..........................................................................................52 Phone, Calendar, and Web Conferencing............................ 55 Smart Garage..................................................................................57

Driving..................................................................60 Starting and Powering Off........................................................ 60 Steering Wheel...............................................................................62 Mirrors............................................................................................... 66 Shifting..............................................................................................68 Lights.................................................................................................. 71 Wipers and Washers....................................................................74 Braking and Stopping..................................................................75 Park Assist....................................................................................... 78 Vehicle Hold....................................................................................80 Traction Control..............................................................................81 Acceleration Modes......................................................................82 Driver Profiles.................................................................................83 Active Hood.................................................................................... 85 Trip Information.............................................................................86 Rear Facing Camera(s)............................................................... 87 Pedestrian Warning System......................................................88 Towing and Accessories.............................................................89

Autopilot..............................................................96 About Autopilot............................................................................ 96

Contents

Autopilot Features........................................................................98 Traffic Light and Stop Sign Control......................................105 Autopark.......................................................................................... 112 Summon...........................................................................................114 Smart Summon..............................................................................116 Limitations and Warnings..........................................................118

Active Safety Features................................. 124 Lane Assist.....................................................................................124 Collision Avoidance Assist........................................................127 Speed Assist..................................................................................130 Cabin Camera.................................................................................131

Dashcam, Sentry, and Security..................132 Safety & Security Settings........................................................132 Dashcam......................................................................................... 134 Sentry Mode.................................................................................. 136 USB Drive Requirements for Recording Videos.............. 138

Climate................................................................139 Operating Climate Controls.....................................................139 Adjusting the Front and Rear Vents.....................................143 Cold Weather Best Practices.................................................. 145 Hot Weather Best Practices....................................................148

Navigation and Entertainment..................149 Maps and Navigation................................................................. 149 Media................................................................................................155 Theater, Arcade, and Toybox...................................................157

Charging and Energy Consumption........160 Electric Vehicle Components................................................. 160 High Voltage Battery Information.........................................162 Charging Instructions................................................................ 164 Scheduled Charging and Scheduled Departure..............169 Getting Maximum Range..........................................................170

Maintenance......................................................172 Software Updates........................................................................172 Maintenance Service Intervals................................................174 Tire Care and Maintenance...................................................... 176 Cleaning.......................................................................................... 183 Windshield Wiper Blades, Jets and Fluid...........................187 Jacking and Lifting..................................................................... 189 Parts and Accessories............................................................... 190 Do It Yourself Maintenance......................................................192

Specifications...................................................193 Identification Labels...................................................................193 Vehicle Loading............................................................................194 Dimensions.....................................................................................195 Subsystems....................................................................................197

Contents

Wheels and Tires........................................................................200

Roadside Assistance.................................... 203 Instructions for Transporters................................................. 203

In Case of Emergency..................................206 Contacting Tesla Roadside Assistance.............................. 206 Running Out of Range..............................................................207 Opening the Front Trunk with No Power..........................208 Jump Starting...............................................................................210 Opening Doors with No Power................................................211

Troubleshooting...............................................212 Troubleshooting Alerts..............................................................212

Consumer Information.................................266 About this Owner Information.............................................. 266 Feature Availability Statement..............................................268 Disclaimers....................................................................................269 Reporting Safety Defects........................................................270 Certification Conformity........................................................... 271

Index....................................................................272

Exterior

1. Key card sensor (Keys on page 19)

2. Charge port (Charging Instructions on page 164)

3. Hood/front trunk (Front Trunk on page 28)

4. Front tow eye cover (Instructions for Transporters on page 203)

5. Rear trunk (Rear Trunk on page 26)

6. Rear view camera (Rear Facing Camera(s) on page 87)

7. Rear tow eye cover (Instructions for Transporters on page 203)

For Autopilot camera locations, see Cameras on page 18.

Overview 3

Interior Overview

NOTE: On RHD (Right Hand Drive) vehicles, the controls illustrated throughout the Owner's Manual are arranged  similarly, but are mirrored on the right side of the vehicle.

1. Door open button (Opening Doors from the Interior on page 22)

2. Blind spot indicator (Blind Spot Indicator on page 125)

3. Left scroll wheel (Left Scroll Wheel (Multifunction) on page 64)

4. Horn (Horn on page 65)

5. Right scroll wheel (Right Scroll Wheel (Autopilot) on page 64)

6. Touchscreen (Touchscreen on page 6)

7. Secondary drive mode selector (P R N D) (Shift Using the Overhead Console on page 68) 8. Cabin camera (Cabin Camera on page 131)

9. Hazards (Hazards on page 73)

10. Dome lights (Lights on page 71)

11. Accent lights (Lights on page 71)

12. Climate control vent (see Operating Climate Controls on page 139)

13. Power window switches (Windows on page 25)

14. Manual door release (Opening Doors from the Interior on page 22)

15. Brake pedal (Braking and Stopping on page 75)

16. Accelerator pedal (Regenerative Braking on page 76)

17. Wireless phone charger (Wireless Phone Chargers on page 11) and key card reader (Keys on page 19) 18. Center console (Center Console on page 30)

19. Glovebox (Glovebox on page 30)

4 MODEL 3 Owner's Manual

Interior Overview

1. Rear touchscreen (Rear Touchscreen on page 9)

2. Adjustable climate control vents (Operating Climate Controls on page 139)

3. Seat pocket

4. Cup holders (Rear Console on page 30)

5. USB ports (USB Ports on page 11)

Overview 5

Touchscreen

WARNING: Always pay attention to road and traffic conditions when driving. To minimize driver distraction and  ensure the safety of vehicle occupants as well as other road users, avoid using the touchscreen to adjust  settings while the vehicle is in motion.

Use the touchscreen to control many features that, in traditional cars, are controlled using physical buttons (for  example, adjusting the cabin heating and air conditioning, headlights, etc.). You also use the touchscreen to control  media, navigate, use entertainment features, and customize Model 3 to suit your preferences. For hands-free access  to common touchscreen controls, use voice commands (see Voice Commands on page 16).

If the touchscreen is unresponsive or demonstrates unusual behavior, you can restart it (see Restarting the  Touchscreen on page 8).

NOTE: Illustrations are provided to improve conceptual understanding only. Depending on vehicle options, software  version, market region and regional and language settings, the details displayed on the screen will differ.

1. Status bar: Find car controls and status in the top bar (see Top Status Bar Icons on page 7).

2. Navigation: Change the orientation of the map, find or navigate to a destination, and change navigation settings  (see Maps and Navigation on page 149).

3. Car status: This area dynamically displays the current status of Model 3 as you drive, park, open doors, turn lights  on, etc. Monitor this area when driving as it displays important information such as driving speed and warning  messages (see Car Status on page 13). When the vehicle is in Park, you can open the trunks or charge port door.  This area also houses shortcut "cards" for Media, tire pressures, and Trip Information.

6 MODEL 3 Owner's Manual

Touchscreen

When full self-driving is enabled (if equipped), the car status area displays visualizations of the road and your  vehicle's surroundings. You can expand/condense the visualization by dragging the car status area from side to  side. Expanding the visualization displays more details about the roadway and its surroundings, including road  markings, stop lights, and objects (such as trash cans and poles).

4. Drive mode strip: Use to shift into Park, Reverse, Neutral, or Drive. Swipe from the edge of the touchscreen  towards the passenger, to bring up the drive mode strip. (See Shifting on page 68.)

5. Controls: Control various features and customize Model 3 to suit your preferences. The Controls screen appears  over the map. Touch an option on the Controls screen to display the various settings and preferences associated  with the chosen option.

To search for a specific setting, touch Search at the top of the Controls screen. Make changes directly from the  result or touch the link to jump to that option in Controls.

When an information icon displays beside a specific setting, touch it to display a popup that  provides helpful details about the associated setting.

NOTE: Many vehicle controls, settings, and preferences (such as climate, media, and navigation) can be adjusted  hands-free using voice commands (see Voice Commands on page 16).

NOTE: You can send touchscreen feedback to Tesla by long-pressing this icon.

6. Climate controls (driver): Use the left and right arrows to decrease/increase cabin temperature. Touch Split on  the popup to display separate controls for the driver and passenger. Touch the temperature icon to customize  climate control settings (see Operating Climate Controls on page 139). The passenger climate controls display  when temperature controls have been Split to provide separate controls for the driver and passenger.

7. Media player shortcut: When using the media player app, swipe downwards and the media player shortcut icon  appears in the bottom bar.

8. My Apps: For one-touch access to frequently used apps and controls, you can choose what displays here. See  Customizing My Apps on page 7.

9. App Launcher: Touch the app launcher to open the app tray. Then touch any app to open it. The app you choose  displays on top of the map. To close an app, drag it downward.

10. Recent App(s): Displays the most recently used app(s). The number of recent apps displayed here depends on  how many apps have been added to My Apps. If you add the maximum number of apps to My Apps, only the  most recent app displays.

11. Climate controls (passenger): Displays when temperature controls have been Split to provide separate controls  for the driver and passenger.

12. Volume Control: Controls the volume of media player and phone calls (see Volume Controls on page 155). The  volume of navigation instructions is controlled separately (see Maps and Navigation on page 149).

Customizing My Apps

For one-touch access to commonly used apps and  controls, you can customize what displays in the My  Apps area on the touchscreen's bottom bar:

1. Enter customization mode by touching and holding  any app or control in the My Apps area. If this area is  empty, touch the App Launcher.

2. Drag any app or control from the app tray onto the  My Apps area in the bottom bar.

NOTE: Seat heaters selected from the app tray  appear next to the temperature, instead of in the My  Apps area.

NOTE: When you've added the maximum number of  apps or controls to My Apps, adding an additional app  removes the rightmost app.

NOTE: Remove an app or control from the My Apps area  by touching and holding, then touching its associated  "X".

Top Status Bar Icons

Touch to lock/unlock all doors and

trunks.

Using This Owner's Manual 7

Touchscreen

4:20 pm Your vehicle automatically updates the  time. If the time is incorrect, confirm

your vehicle has internet and GPS

connectivity with the latest software.

Displays on the touchscreen status bar

only when Model 3 detects a

programmed HomeLink within range,

and the touchscreen is not already

displaying the HomeLink screen or

popup. See Smart Garage on page 57.

Displays on the touchscreen status bar

only when Model 3 is parked. Add,

configure (including Valet Mode and Use

Easy Entry), or quickly switch driver

profiles. Driver profiles can also be

accessed from the top of any Controls

screen. See Driver Profiles on page 83.

Available when Model 3 is parked, touch

to manually enable or disable Sentry

Mode for the current drive cycle. To

automatically turn Sentry Mode on (or

off) every time you leave your vehicle,

enable the setting from Controls >

Safety > Sentry Mode. See Sentry Mode

on page 136 for more information.

NOTE: If you turn Sentry Mode on or off

from Controls > Sentry Mode, the

shortcuts on the vehicle's touchscreen

and mobile app will only work for the

current drive cycle.

NOTE: For some vehicles manufactured

after approximately November 1, 2021,

the center console USB ports may only

support charging devices. Use the USB

port inside the glove box for all other

functions.

Displays when Model 3 is connected to a

Wi-Fi network.

Displays when Model 3 cellular

connectivity is very low or unavailable.

Touch this icon for quick access to Wi-Fi

settings.

Status of the front passenger airbag (see

#unique_44 on page ).

Appears when your vehicle's GPS

location is actively being accessed in the

Tesla mobile app by the owner, an added

driver, or a third party app you're using.

To disable, navigate to Safety > Allow

Mobile Access on the touchscreen.

Popup Messages and Vehicle Alerts

Popup messages appear at the bottom of the  touchscreen. For example, a seat belt reminder appears  if a seat belt is unfastened in an occupied seat, an alert  appears to notify you of an incoming phone call, a text  message appears (when applicable), and voice  commands appear when in use. If applicable, touch  options from these popup messages (for example,  accept/decline a phone call, choose an option from the  headlight menu, etc.). To dismiss a popup message,  swipe it downward.

If an alert appears on your vehicle's touchscreen, touch  Learn More for more details regarding the alert and how  it can be resolved. You can view a list of vehicle alerts  and notifications by touching the bell icon at the top of  Controls.

NOTE: Not all alerts provide additional information at  this time.

Restarting the Touchscreen

You can restart your touchscreen if it is unresponsive or  demonstrates unusual behavior.

WARNING: Only restart the touchscreen while the  vehicle is stopped and in Park. The car status  display, safety warnings, backup camera, etc. will  not be visible during the restart.

1. Shift into Park.

2. Hold down both scroll buttons on the steering wheel  until the touchscreen turns black. Pressing the brake  pedal while holding down the scroll buttons does not  have any impact and is not required.

8 MODEL 3 Owner's Manual

3. After a few seconds, the Tesla logo appears. Wait  approximately 30 seconds for the touchscreen to  restart. If the touchscreen is still unresponsive or  demonstrating unusual behavior after a few minutes,  try power cycling the vehicle (if possible). See Power  Cycling the Vehicle on page 61.

NOTE: Pressing the scroll buttons only restarts the  touchscreen. It does not restart any other vehicle  component and does not power Model 3 off and on.

Rear Touchscreen

The rear touchscreen provides rear passengers with  access to:

1. Power: Touch to turn the rear climate control system  on or off.

2. Rear fan speed: Touch to adjust fan speed.

3. Rear fan: Touch to turn the rear fan on or off, to  adjust fan speed and control the direction of air flow from the rear vents (see Adjusting the Front and Rear  Vents on page 143).

4. Seats: Control rear seat heaters and move the front  passenger seat forward/rearward using the arrows.

5. Media: Play, pause, skip or rewind through the  currently playing song (see Media on page 155).

6. Video: Access video streaming services.

7. Settings: Touch to pair up to two sets of Bluetooth  headphones, change the brightness or clean the  display.

NOTE: You can also use the front touchscreen to pair  Bluetooth headphones to the rear display by  launching the Rear Screen app.

8. Volume: Touch to adjust the volume.

9. Air wave preset: Touch to direct the air flow to the  passenger head or torso quickly. Direct the air flow exactly where you want by touching and moving the  air wave on the touchscreen.

Touchscreen

NOTE: You can also use the front touchscreen to adjust  climate settings in the rear cabin (see Adjusting Climate  Control Settings on page 139).

NOTE: Adjusting the media and volume controls also  adjusts the front cabin settings.

Control the rear screen from the front

To control the rear touchscreen from the

front, open the rear screen remote control

app in the bottom bar on the front

touchscreen. Besides audio, video and

climate controls, you can lock the rear

display in the app or by touching Controls

> Display > Lock Rear Display.

Customizing Display and Sound  Settings

Touch Controls > Display to adjust display settings to  suit your preferences:

• Appearance: Customize the display to be Dark or  Light. When set to Auto, the brightness changes  automatically based on ambient lighting conditions.

• Brightness: Drag the slider to manually control the  brightness level. If Display Mode is set to Auto, the  touchscreen further adjusts based on both the  ambient lighting conditions and your brightness  preference. Model 3 remembers your chosen  brightness preference and adjusts the touchscreen  accordingly.

• Screen Clean Mode: When enabled, your touchscreen  darkens and temporarily disables to facilitate  cleaning. Follow the onscreen instructions to exit  Screen Clean Mode.

• Touchscreen Language: Select the language that the  touchscreen displays.

NOTE: Model 3 must be in Park to change the  language. When you change the language, you  experience a brief delay as Model 3 shuts down and  restarts the touchscreen.

• Voice Recognition Language: Choose the language  to be used for voice commands.

• Voice Navigation Language: Choose the language  that the navigation system uses for spoken  instructions.

NOTE: For languages that require a download, select  the language in the dropdown list to initiate the  download (Wi-Fi connection required).

• Text size: Select between Standard and Large to  customize the text size on your vehicle's touchscreen.

• Time: Choose to display time in either 12 or 24 hour  format.

Overview 9

Touchscreen

• Energy Display: Choose to display remaining energy

and charging units as either a percentage of battery

energy remaining, or as an estimate of the distance

you can drive.

NOTE: When anticipating when you need to charge,

use energy estimate as a general guideline only. Many

factors have an impact on energy consumption. See

Factors Affecting Energy Consumption on page 170.

• Distance: Choose to display measurements in metric

(kilometers, centimeters, etc.) or imperial (miles,

inches, etc.) units.

• Temperature: Choose to display temperature using

Fahrenheit or Celsius.

• Tire Pressure: Choose to display tire pressures using

BAR or PSI.

In addition to customizing the display, you can enable

Joe Mode to reduce the volume of all chimes that are

not related to critical safety issues. Touch Controls >

Safety > Joe Mode to enable.

Naming your Vehicle

To further personalize your vehicle, you can name it.

Touch Controls > Software > Name Your Vehicle located

on the right side of the touchscreen below the image of

Model 3. If your vehicle already has a name, touch the

existing name to change it. Enter the new name in the

popup and touch Save. The name of your Model 3 also

appears in the Tesla mobile app.

Erasing Personal Data

You can erase all personal data (saved addresses, music

favorites, HomeLink programming, etc.) and restore all

customized settings to their factory defaults. This is

useful when transferring ownership of Model 3. Touch

Controls > Service > Factory Reset. Before erasing,

Model 3 verifies your credentials by prompting you to

enter the user name and password associated with your

Tesla account.

10 MODEL 3 Owner's Manual

In addition to storage compartments and cup holders  (see Interior Storage on page 30), the Model 3 interior  supports various electronics such as USB ports, wireless  phone chargers, and a 12V power socket.

USB Ports

Model 3 has four USB ports. The three USB-C ports  output up to approximately 42W or two ports up to  approximately 65W.

• One USB-C port is located in the rear compartment  of the center console used to charge USB devices.

• Two USB-C ports are located below the rear  touchscreen that can be used to charge USB devices.

• One USB-A port is located inside the glovebox. This  port is equipped with a USB flash drive. The secure  location of this USB port makes it ideal for saving  Sentry Mode and Dashcam video footage. It also  supports audio files from a phone or USB device (see  Playing Media from Devices on page 156).

Interior Electronics

See USB Drive Requirements for Recording Videos on  page 138 for information about formatting USB flash drives.

NOTE: Power is available whenever the vehicle is  considered "awake". The vehicle may be awake for many  reasons. For example, when using features such as  Summon, or when features such as Keep Climate On,  Dog Mode, Camp Mode, or Sentry Mode are enabled.  The vehicle is also awake whenever the low voltage  battery is being charged or is in use, during high voltage  charging, when the vehicle is communicating with the  mobile app, etc. Leaving an accessory plugged in does  not deplete the low voltage battery.

NOTE: Use USB 3.0 compliant cables to connect a  device to a USB port. Using non-compliant cables can  result in slower charging, potential connection problems  or degraded performance.

NOTE: Do not connect multiple devices using a USB  hub. This can prevent connected devices from charging  or from being recognized by Media Player, Sentry Mode,  Dashcam, etc.

Wireless Phone Chargers

Wireless phone chargers are integrated into the front  console, providing up to 15W of power to charge Qi enabled phones. To charge your phone, place it on one  of the two charge pads. The phone must be in direct  contact with the wireless charger. Do not place objects  between the phone and the charger (for example, credit  cards, key cards, coins, keys, metal objects, etc.). Your  device may feel warm while charging, but this is a  normal effect of inductive charging.

CAUTION: Remove NFC cards (for example, the  vehicle key card, credit cards, or hotel key) from  integrated phone cases before charging your  phone to avoid damage to the card.

When placed on the wireless charger, your phone  charges whenever the vehicle is powered on (the  touchscreen is on and you are in the vehicle). Your  phone does not charge when you leave the vehicle  unless a feature, such as Keep Climate On, Dog Mode,  Camp Mode, or Sentry Mode is enabled.

Overview 11

Interior Electronics

NOTE: The wireless phone charger may not work if your

phone case is too thick or is made of metal. Try

removing the phone from its case before placing it on

the charger.

NOTE: The wireless phone charger does not charge if

the vehicle's high voltage Battery is discharged.

Low Voltage Power Socket

Your Model 3 has a power socket located in the center

console's compartment. To access the low voltage

socket, open the front compartment of the center

console.

The power socket is suitable for accessories requiring up

to 12A continuous draw (16A peak).

NOTE: Power is available whenever the vehicle is

considered "awake". The vehicle may be awake for many

reasons. For example, when using features such as

Summon, or when features such as Cabin Overheat

Protection, Keep Climate On, Dog Mode, Camp Mode,

Sentry Mode, etc. are enabled. The vehicle is also awake

whenever the low voltage battery is being charged or is

in use, during high voltage charging, when the vehicle is

communicating with the mobile app, etc. Leaving an

accessory plugged in does not deplete the low voltage

battery.

WARNING: The power socket and an accessory’s

connector can become hot.

WARNING: To prevent excessive interference with

the vehicle’s electronics, Tesla recommends that

you do not plug any non-Tesla accessories,

including power inverters, into the low voltage

power socket. However, if you do use a non-Tesla

accessory and notice any malfunctions or

unexpected behavior, such as indicator lights, alert

messages, or excessive heat from the accessory,

unplug the accessory from the low voltage power

socket immediately.

CAUTION: Do not attempt to jump start Model 3

using the low voltage power socket. Doing so can

result in damage.

12 MODEL 3 Owner's Manual

Overview

Cards

Car Status

The touchscreen displays the status of Model 3 at all  times. What you see depends on whether the vehicle is:

• Parked (shown below).

• Driving (see Driving Status on page 14).

• Charging (see Charging Status on page 165).

When Model 3 is parked, the status area shows the drive  mode, estimated range, and an overhead view of the car  with buttons you can touch to open the trunks and  charge port door. When you press the brake, Model 3  powers up and indicator lights flash briefly. Unless an  indicator light applies to the current situation (for  example, a seat belt is not fastened), it should turn off. If  an indicator light fails to turn on or off, contact Tesla.

NOTE: The following image is provided for

demonstration purposes only. Depending on vehicle  options, software version, and market region, the  information displayed may be slightly different.

The bottom of the car status display also shows shortcut  "cards" for quick access to Media, tire pressure data, trip  information, and more. Swipe the cards to the left or  right to customize your cards shortcuts.

Indicator Lights

The following indicator lights illuminate to advise you or  alert you of a specific status or condition.

A brake system fault is detected or the

brake fluid level is low. See Braking and

Stopping on page 75. Contact Tesla

immediately.

A brake booster fault has been

detected. See Braking and Stopping on

page 75.

An ABS (Anti-lock Braking System)

fault is detected. See Braking and

Stopping on page 75. Contact Tesla

immediately.

A parking brake fault is detected.

Contact Tesla. See Parking Brake on

page 77.

The parking brake is manually applied.

See Parking Brake on page 77.

Tire pressure warning. The pressure of a

tire is out of range. If a fault with the

Tire Pressure Monitoring System

(TPMS) is detected, the indicator

flashes. For a TPMS fault, contact Tesla.

See Tire Care and Maintenance on page

176.

A seat belt for an occupied seat is not

fastened. See Seat Belts on page 34.

Airbag safety. If this indicator does not

flash on briefly when Model 3 prepares

to drive, or if it remains on, contact

Tesla immediately. See #unique_68 on

page .

Overview 13

Car Status

The rear fog indicator displays on the  touchscreen whenever rear fog lights  are on.

Parking lights are on (side marker

lights, tail lights, and license plate

lights) . See Lights on page 71.

Low beam headlights are on.

High beam headlights are on and Auto  High Beam is disabled or currently  unavailable.

Auto High Beam is enabled and high  beams are on. Model 3 is ready to turn  off the high beams if light is detected.  See High Beam Headlights on page 71.

Auto High Beam is enabled but high  beams are not on because light is

detected in front of Model 3. When light  is no longer detected, high beams

automatically turn back on. See High  Beam Headlights on page 71.

This indicator flashes amber when the  electronic stability control systems are  actively minimizing wheel spin by

controlling brake pressure and motor  power. See Traction Control on page 81.  If this indicator remains on, a fault is  detected and you should immediately  contact Tesla.

Electronic stability control systems are  no longer minimizing wheel spin. On a  Rear Wheel Drive vehicle, the traction  control system has been turned off, or

on an All-Wheel Drive vehicle, Slip Start  has been enabled. See Traction Control  on page 81.

Vehicle Hold is actively applying the  brakes. See Vehicle Hold on page 80.

A door or trunk is open. See Doors on  page 22, Rear Trunk on page 26, or  Front Trunk on page 28.

A blue snowflake appears when some

of the energy stored in the Battery may

not be available due to cold weather

conditions. During these cold weather

conditions, charging rates may also be

limited. If Model 3 is plugged in, you

can heat your Battery by turning on

climate control with the mobile app.

The snowflake disappears when the

Battery is sufficiently warm.

Appears when regenerative braking is

limited. See Regenerative Braking on

page 76 for more information.

Vehicle power is currently being limited

because the energy remaining in the

Battery is low, the vehicle's systems are

being heated or cooled, or an error is

detected by the drive inverter .

See Popup Messages and Vehicle Alerts on page 8 for  more information about alert popups on your vehicle's  touchscreen.

Driving Status

When Model 3 is driving (or ready to drive), the  touchscreen shows your current driving status and a  real-time visualization of the road as detected by the  Autopilot components (see Cameras on page 18). The  visualization automatically zooms in and out to better  utilize touchscreen space and inform you when a vehicle  is detected in your blind spot.

NOTE: The following illustration is provided for  demonstration purposes only. Depending on vehicle  options, software version, and market region, the  information displayed may be slightly different.

14 MODEL 3 Owner's Manual

NOTE: To display more details about the roadway and its  surroundings, such as road markings, stop lights, objects  (such as trash cans and poles), etc., touch Controls >  Autopilot > Full Self-Driving Visualization Preview (if  equipped).

1. The power meter displays real-time power usage.  During acceleration, the bar fills to the right to  represent power being used. During deceleration  (when Model 3 is moving and you release your foot  from the accelerator pedal), the bar fills to the left  with a green color to represent power being fed back  to the Battery by regenerative braking (see  Regenerative Braking on page 76).

2. When Autosteer is available but you haven't activated  it, the icon is gray. When Autosteer is actively  steering Model 3, the icon is blue (see Autosteer on  page 99).

3. Currently selected drive mode: Park, Reverse, Neutral,  or Drive.

4. Driving speed.

5. Other cars detected on the road (as applicable).

6. Your Model 3. Colored lines radiate from the image of  your Model 3 as objects are detected (other  motorists, guard rails, etc.). The location of the lines  correspond to the location of the detected object.  The color of the lines (white, yellow, orange, or red)  represents the object's proximity to Model 3, with

Car Status

white being the farthest and red being very close and  requiring your immediate attention. See Lane Assist  on page 124.

7. When Autosteer is active and detecting the driving  lane, the lane is highlighted in blue (see Autosteer on  page 99).

NOTE: If Navigate on Autopilot is active, the driving  lane displays as a single blue line in front of Model 3  (see Navigate on Autopilot on page 100).

8. The speed limit that is currently being detected by  Speed Assist (see Speed Assist on page 130).

NOTE: On roads where the map data determines that  a conditional speed limit exists (for example, a speed  limit based on time of day or weather conditions), a  second speed limit displays. It is the driver's  responsibility to determine whether the conditional  speed limit is currently in effect and adjust the  driving speed accordingly.

NOTE: The icon associated with the detected speed  limit reflects the style of speed limit signs used in  your market region.

9. Total estimated driving distance (or energy) available.  Touch the displayed value to change how available  energy is displayed. You can toggle between driving  distance and percentage of battery energy remaining.  You can also change how energy is displayed by  touching Controls > Display > Energy Display.

NOTE: When anticipating when you need to charge,  use range estimates as a general guideline only.

10. The set cruising speed. When Traffic-Aware Cruise  Control is available but you haven't set a cruising  speed, the number is gray (see Traffic-Aware Cruise  Control on page 98).

WARNING: Pay attention to important alert  messages that display at the bottom of the car  status area of the touchscreen. Ignoring these  messages can result in serious injury or death.

WARNING: Although the touchscreen shows  surrounding traffic, some vehicles may not be  displayed. Never rely on the touchscreen to  determine if a vehicle is present (for example, in  your blind spot). Always use your mirrors and  perform shoulder checks.

Overview 15

Voice Commands

NOTE: Voice commands are not available in all  languages. To choose the language you want to use for  voice commands, touch Controls > Display > Voice  Recognition Language.

Use voice commands to easily control settings and  preferences without using the touchscreen. Voice  commands are designed to understand natural requests.  Use them to:

• Adjust climate preferences.

• Tweak the windshield wiper speed and frequency. • Control various aspects of your vehicle.

• Navigate to a location.

• Call a contact.

• Interact with apps and settings.

To initiate a voice command, fully press the microphone  button on the right side of the steering wheel. When a  chime sounds, make your request, then Model 3 provides  onscreen responses.

Examples of Voice Commands

Here is a list of example voice commands. This is not an  exhaustive list. Tesla is constantly working to improve  voice commands.

Climate Controls

Adjust your climate preferences using voice commands: • "Make it cooler"

• "Make it warmer"

• "Turn on/off the driver's seat heater"

• "Cool down the passenger"

• "Direct airflow to my face"

• "Sync climate"

• "Increase/decrease the fan speed"

• "Turn on/off rear defroster"

• "Set the temperature/fan..."

• "Turn on recirculate"

Windshield Wipers

Update the windshield wiper speed and frequency  based on changing road and weather conditions:

• "Speed up the wipers"

• "Increase/decrease windshield wiper speed by..." • "Turn on/off the wipers"

Vehicle Controls

You can also modify various controls in your vehicle  using voice commands:

NOTE: Your vehicle must be in Park to enable some  voice commands (such as Sentry Mode, Dog Mode, etc.).

• "Sentry Mode on/off"

• "Keep my car safe"

• "Lock/unlock the doors"

• "Turn on Dog Mode"

• "Fold/unfold the mirrors"

• "Open/close charge port"

• "Start/stop charging"

• "Open service settings"

• "Open the glovebox"

Navigation

To search for, or navigate to, a location, say:

• "Where is [location]?"

• "Drive to [location]"

• "Navigate to [location]"

• "Show nearby Superchargers"

• "I'm feeling hungry/lucky" (see Maps and Navigation  on page 149).

• "Stop navigation"

• "Mute voice guidance"

16 MODEL 3 Owner's Manual

If you have defined a navigation address for your home  or work locations, you can use a voice command to  navigate there by saying "Navigate home" or "Take me  to work".

Contacts

To call or text a contact on your Bluetooth-connected  phone (see Phone, Calendar, and Web Conferencing on  page 55), say:

• "Call [contact name/phone number]"

• "Text [contact name/phone number]"

Media

To listen to and adjust your media preferences, say:

• "Listen to [song name]"

• "Lower/raise the volume"

• "Skip to next"

• "Pause/play song"

• "Change the source to [media source]"

To improve voice command recognition accuracy,  provide multiple cues in your command, such as artist  and song.

Apps and Settings

Easily navigate through your apps and settings using  voice commands:

• "Open [Toybox/browser/theater/phone]" • "Search for..."

• "The screen is too bright"

• "Show me the Owner's Manual"

Some apps and settings are only accessible while in  Park.

You can also file a bug report by saying "Report" ,  "Feedback", or "Bug report".

For more information on voice commands, go to https:// www.tesla.com/support/voice-commands.

NOTE: To support ongoing quality improvements, Tesla  captures and processes voice command transcriptions  (such as “set the temperature…”). Audio voice  recordings are not collected, and transcriptions are not  associated with your Tesla account or with your vehicle's  identification number. To further protect your privacy,  voice commands containing personal data are not  captured (such as "Navigate to..." or "Make a call to...").

Voice Commands

Overview 17

Cameras

Your Model 3 includes the following components that  actively monitor the surrounding area:

• A camera is mounted above the rear license plate. • A camera is mounted in each door pillar.

• Two cameras are mounted to the windshield above  the rear view mirror.

• A camera is mounted to each front fender.

Model 3 is also equipped with high precision  electronically-assisted braking and steering systems.

Cabin Camera

Your Model 3 is equipped with a cabin camera located  above the rear view mirror. For more information, see  Cabin Camera on page 131.

Drive to Calibrate Cameras

Model 3 must maneuver with precision when Autopilot  features are being used. Therefore, before some features  can be used for the first time or after some types of  service repairs, cameras must complete a self-calibration  process. For your convenience, the touchscreen displays  a progress indicator.

When calibration is complete, Autopilot features are  available for use. Calibration typically completes after  driving 32-40 km, but the distance varies depending on  road and environmental conditions. For example,  calibration completes quicker when driving on a straight  road with multiple lanes (such as a controlled-access  highway), with highly-visible lane markings (in the  driving lane as well as the adjacent lanes). Contact Tesla  only if your Model 3 has not completed the calibration  process after driving 160 km in the described conditions.

If a camera has shifted from its calibrated position (for  example, the camera or windshield was replaced), you  must clear the calibration. To do so, touch Controls >  Service > Camera Calibration > Clear Calibration. When  the calibration is cleared, Model 3 repeats the calibration  process. While this helps re-calibrate the cameras in  many cases, Clear Calibration may not resolve all  camera and sensor concerns.

NOTE: To calibrate, cameras require highly-visible lane  markings in both the driving lane and adjacent lanes (at  least two lanes over on each side of the vehicle). For  best results, drive in the middle lane of a multi-lane  highway (ideally with at least five lanes) that has clear  lane markings and minimal traffic.

NOTE: If you attempt to use a feature that is not  available until the calibration process is complete, the  feature is disabled and the touchscreen displays a  message.

NOTE: Model 3 must repeat the calibration process if the  cameras are serviced by Tesla, and in some cases, after a  software update.

Keeping Cameras Free of

Obstructions

Ensure all cameras are clean and free of obstructions  before each drive and before using Autopilot features  (see Cleaning a Camera on page 183). Dirty cameras and  sensors (if equipped), as well as environmental  conditions such as rain and faded lane markings, can  affect Autopilot performance. If a camera is obstructed  or blinded, Model 3 displays a message on the  touchscreen and Autopilot features may not be  available. For more information on specific alerts, see  Troubleshooting Alerts on page 212.

Condensation can form inside the camera enclosures,  especially if you park your vehicle outside in cold or wet  conditions. The touchscreen may display an alert stating  that a camera is blocked and that some or all Autopilot  features may be temporarily restricted until the camera  vision is clear. To proactively dry the condensation,  precondition the cabin by setting it to a warm  temperature, turning the windshield defroster on, and  directing the front air vents toward the door pillars (see  Mobile App on page 49).

18 MODEL 3 Owner's Manual

Types of Keys

Model 3 supports the following types of keys:

• Phone key - You can set up your personal phone as a  "phone key" that communicates with Model 3 using  Bluetooth. A phone key supports automatic locking  and unlocking.

• Key card - Tesla provides a key card that

communicates with Model 3 using short range radio frequency identification (RFID) signals. The key card  is used to "authenticate" phone keys to work with  Model 3 and to add or remove other keys. Unlike the  phone key, the key card does not support automatic  locking and unlocking. In situations where your phone  key has a dead battery, or is lost or stolen, use your  key card to unlock, drive, and lock Model 3.

Model 3 supports a total of 19 keys, which can include  phone keys and key cards.

CAUTION: Remember to bring a key with you  when you drive. Although you can drive Model 3  away from its key, you will be unable to power it  back on after it powers off.

Phone Key

CAUTION: Do not leave your paired phone in your  vehicle (for example, if you are hiking or at the  beach). If you must leave your phone in the  vehicle, disable Bluetooth and/or turn the phone  off.

Using your phone as a key is a convenient way to access  your Model 3. As you approach, your phone's Bluetooth  signal is detected and the doors unlock when you pull a

door handle. Likewise, when you exit and walk away with  the phone key, doors automatically lock (provided the  Walk-Away Door Lock feature is turned on; see Walk Away Door Lock on page 23).

Before you can use a phone to access Model 3, follow  these steps to authenticate it:

1. Download the Tesla mobile app to your phone.

2. Log into the Tesla mobile app using your Tesla  account username and password.

NOTE: You must remain logged in to your Tesla  account to use your phone to access Model 3.

NOTE: If multiple vehicles are linked to your Tesla  account, you must ensure that the vehicle you want  the mobile app to access is currently selected on the  mobile app.

3. Ensure:

◦ Your phone's general Bluetooth settings are  enabled.

Keys

◦ Bluetooth is enabled within your phone's settings  for the Tesla mobile app. For example, on your  phone, navigate to Settings, choose the Tesla  mobile app, and ensure the Bluetooth setting is  turned on.

◦ Access to your location is enabled. Open the Tesla  mobile app in your phone's settings and select  Location > Always. For the best experience, keep  the mobile app running in the background.

◦ Allow Mobile Access is enabled on the vehicle  touchscreen (Controls > Safety > Allow Mobile  Access).

NOTE: Model 3 communicates with your phone using  Bluetooth. Keep in mind that your phone must have  enough battery power to run Bluetooth and that  many phones disable Bluetooth when the battery is  low.

4. While inside or near the vehicle, open the Tesla  mobile app and touch Set Up Phone Key on the main  screen, or navigate to Security > Set Up Phone Key.  Follow the prompts on the mobile app and vehicle  touchscreen to set up your phone key.

To view a list of keys that can currently access Model 3,  or to remove a phone key, touch Controls > Locks (see  Managing Keys on page 20).

Model 3 can connect to three phone keys

simultaneously. Therefore, if more than three phone keys  are detected and you want to authenticate or pair a  different phone, move the other connected phone key(s)  out of range or turn off its Bluetooth setting.

Once a phone has been authenticated, it no longer  requires an internet connection to be used as a phone  key for Model 3. However, to use the phone hands-free,  access your phone's contacts, play media from it, etc.,  you must also pair it and connect it as a Bluetooth  device (see Bluetooth on page 52).

Some smartphones with NFC capability can be used to  lock/unlock your vehicle, just like using a key card.  Ensure the Tesla mobile app is correctly paired to your  vehicle and enable the NFC function on your phone.  Once enabled, simply hold the phone to the driver's side  door pillar to lock or unlock the door. Refer to your  smartphone's instructions for specific information on  how to do this.

Key Card

Tesla provides you with two Model 3 key cards, designed  to fit in your wallet.

Opening and Closing 19

Keys

To use a key card to unlock or lock Model 3, position the  card as shown and tap it against the card reader located  approximately one third the way up of the driver's side  door pillar. When Model 3 detects the key card, the  exterior lights flash, the mirrors unfold or fold (if Fold  Mirrors is on), the horn sounds (if Lock Confirmation Sound is on), and the doors unlock or lock.

NOTE: You may need to physically touch the wireless  phone charger or driver's side door pillar with the key  card, and you may need to hold it against the  transmitter for one or two seconds.

Once inside, power up Model 3 by pressing the brake  pedal within two minutes of scanning the key card (see  Starting and Powering Off on page 60). If you wait  longer than two minutes, you must re-authenticate by  placing the key card near the card reader located in the  wireless phone charger on the center console. When  your key card is detected, your two minute  authentication period restarts.

NOTE: If enabled, Walk-Away Door Lock (see Walk Away Door Lock on page 23) operates only when you  walk away using a phone key. When you walk away  carrying your key card, Model 3 does not automatically  unlock/lock.

Managing Keys

To display a list of all keys that can access your Model 3,  touch Controls > Locks. An icon displays next to each  key to indicate whether the key is a phone key or key  card. Use this list to manage keys that have access to  your Model 3. To add or delete keys, see Adding and  Removing Keys on page 20.

Model 3 supports up to 19 keys. When you reach this  limit, you must delete a key before adding a new one.

NOTE: You can use the same key for more than one  Model 3. This prevents you from having to deal with  multiple keys when you switch vehicles. If you customize  the name of an authenticated key card on one Model 3  (by touching the pencil icon), any other Model 3 to  which the key card is authenticated also displays the  changed name.

NOTE: If you are leasing your vehicle, contact your  leasing company to add or remove keys.

Adding and Removing Keys

To add a new key:

1. On the touchscreen, touch Controls > Locks > Keys >  Add Key.

2. Scan your new key card on the card reader located  on either wireless phone charger. When you hear a  chime, the new key card is recognized.

3. Scan a key card that has already been paired to the  vehicle to confirm new key pairing.

4. When complete, the key list includes the new key.  Touch the associated pencil icon to customize the  name of the key.

Removing a key:

When you no longer want a key to access Model 3 (for  example, you lost your phone or key card, etc.), follow  these steps to remove it.

1. On the touchscreen, touch Controls > Locks.

2. In the key list, find the key that you would like to  delete and touch its associated trash icon.

3. When prompted, scan an authenticated key on the  card reader to confirm the deletion. When complete,  the key list no longer includes the deleted key.

NOTE: Model 3 requires at least one authenticated key  card at all times. If only one key card remains on the key  list, you cannot delete it.

20 MODEL 3 Owner's Manual

Keys

Replacing Key cards

If you lose a key card, you can purchase replacement

ones on the Tesla Shop. When ready to pair, simply

follow the steps in Adding and Removing Keys on page

20. Remember to remove your old key cards from

Controls > Locks > Keys for security purposes.

In the event you lose both keys cards, schedule a service

appointment through the mobile app to replace and pair

them.

Opening and Closing 21

Doors

Using Exterior Door Handles

Use your thumb to push the wide part of the door  handle. The handle pivots toward you, and you can open  the door by pulling the handle or pulling the edge of the  door.

The handle retracts automatically.

When a door or trunk is open, the

touchscreen displays the Door Open

indicator light.

NOTE: See Cold Weather Best Practices on page 145 to  ensure door handles function properly in cold weather.

WARNING: While using the door handle, take care  to avoid allowing fingers, jewelry, acrylic nails, etc.  from being pinched by the door or door handle  mechanism. Failure to do so may result in damage  or injury.

Opening Doors from the Interior

Model 3 doors are electrically powered. To open a door  while sitting inside, press the button located at the top  of the interior door handle and push the door open.

NOTE: To prevent children from opening the rear doors,  turn on child locks (see Child Locks on page 23).

To open a front door in the unlikely situation when  Model 3 has no power, pull up the manual door release  located in front of the window switches.

You can open a rear door manually in the unlikely  situation in which Model 3 has no power:

22 MODEL 3 Owner's Manual

1. Remove the cover from the bottom of the rear door  pocket.

2. Pull the mechanical release cable forward.

CAUTION: Manual door releases are designed to  be used only in situations when Model 3 has no  power. When Model 3 has power, use the button  located at the top of the interior door handle.

Interior Locking and Unlocking

While sitting inside Model 3, you can lock and unlock all  doors and trunks by touching the lock icon on the  touchscreen.

The icon changes to indicate whether

doors are locked or unlocked.

Walk-Away Door Lock

Doors and trunks can automatically lock when you walk  away carrying your phone key (if ordered after  approximately October 1, 2019). To turn this feature on  or off, touch Controls > Locks > Walk-Away Door Lock.

When the doors lock, the exterior lights flash once and  the mirrors fold (if Fold Mirrors is on). To also hear a  confirmation sound when Model 3 locks, touch Controls  > Locks > Lock Confirmation Sound.

NOTE: Touch Toybox > Boombox > Lock Sound to  customize the lock sound when the vehicle is locked  from the outside (Pedestrian Warning System required).

Model 3 does not automatically lock if:

• You check the Exclude Home checkbox and Model 3  is parked at the location you have designated as  Home. For details on how to designate a location as  Home, see Home, Work, and Favorite Destinations on  page 151.

Doors

• A phone key is detected inside Model 3.

• A door or trunk is not fully closed.

• The phone key's Bluetooth setting is turned off.

• If Model 3 detects an authenticated key for several  minutes after you exit the vehicle and close all doors,  Walk-Away Lock disables and doors do not lock  when you walk away. In this case, you must manually  lock your vehicle until after your next drive.

• The driver does not use the driver door to get out of  the vehicle.

NOTE: It is ultimately your responsibility to ensure your  vehicle is locked, even when Walk-Away Door Lock is  enabled.

Drive Away Locking

Model 3 automatically locks all doors (including the  trunks) when your driving speed exceeds 8 km/h.

Driver Door Unlock Mode

Enabling Controls > Locks > Driver Door Unlock Mode  only unlocks the driver door when you first unlock Model  3. The driver door unlocks only if a key is present on the  driver side of the vehicle and not the passenger side. To  unlock the remaining doors, long press the button  located at the top of the interior driver door handle, use  the touchscreen, mobile app.

Car Left Open Notifications

To receive a mobile notification if a door, trunk and/or  window is left open or if Model 3 is left unlocked  unexpectedly, touch Controls > Locks > Car Left Open  Notifications.

Child Locks

Model 3 has child locks on the rear doors to prevent  them from being opened using the interior release  buttons. On the touchscreen, touch Controls > Locks >  Child Lock. You can choose Both to engage the child  lock on both rear doors, or you can choose Left or Right  to engage it on just a specific door.

WARNING: It is recommended that you turn on  child locks when children are seated in the rear  seats.

Unlock on Park

When you stop Model 3 and engage Park, you can  choose to unlock all doors. To turn this feature on or off, touch Controls > Locks > Unlock on Park.

Opening and Closing 23

Doors

NOTE: If set to OFF, you can unlock all doors by

pressing the Park button a second time after engaging

Park.

24 MODEL 3 Owner's Manual

Opening and Closing

NOTE: It is your responsibility to ensure windows are  closed after locking the vehicle.

Press down on a switch to lower the associated window.  Window switches operate at two levels:

• To lower a window fully, press the switch all the way  down and immediately release.

• To lower a window partially, press the switch gently  and release when the window is where you want it.

NOTE: If a window is fully lowered and you open the  associated door, the window rises slightly. Likewise, if  you fully lower a window while the door is already open,  it stops slightly above the edge of the door. To fully  lower a window while the door is open, press the switch  again.

Similarly, pull a switch to raise the associated window:

• To raise a window fully, pull the switch all the way up  and immediately release.

• To raise a window partially, pull the switch gently and  release when the window is where you want it.

If a window is left open unintentionally, Model 3 can  send a notification to the mobile app (touch Controls >  Locks > Car Left Open Notification, then choose Doors  & Windows).

You can also enable Close Windows on Lock by  touching Controls > Locks > Close Windows on Lock.  When enabled, your vehicle automatically closes the  windows when Model 3 locks.

Windows

NOTE: See Cold Weather Best Practices on page 145 for  information on preparing windows for cold weather.

CAUTION: To avoid damage, windows

automatically lower slightly when you open or  close a door. If you manually raise a window when  the door is open, ensure it is slightly lowered  before closing the door.

WARNING: Before closing a window, it is the  driver’s responsibility to ensure that all occupants,  especially children, do not have any body parts  extended through the window’s opening. Failure to  do so can cause serious injury.

Locking Rear Windows

To prevent passengers from using the rear window  switches, touch Controls > Locks > Window Lock. To  unlock the rear windows, touch Window Lock again.

WARNING: To ensure safety, it is recommended  that you lock the rear window switches whenever  children are seated in the rear seats.

WARNING: Never leave children unattended in  Model 3.

Calibrating Windows

In the unlikely event that a window behaves  unexpectedly (touches the bright molding, fails to open  or close properly, goes down more than normal when  the door opens, etc.), you can calibrate it to potentially  fix the issue.

To calibrate a window:

1. Close the door with the affected window. 2. Sit in the driver's seat and close the driver door.

3. Using the window's switch on the driver's door, raise  the affected window until it stalls.

4. Using the window's switch on the driver's door, lower  the affected window until it stalls.

5. Repeat step 3 and raise the affected window until it  stalls.

The window should now be calibrated. If the issue  continues after attempting the calibration procedure a  couple times, contact Tesla.

UV Index Rating

The roof, windshields, and windows in Model 3 are  excellent at protecting you from UV (ultraviolet) rays.  The glass components score less than 2 on the UV Index  scale. Review your region’s UV Index specifications for  more information. You are still responsible for taking the  necessary precautions for sun protection.

Opening and Closing 25

Rear Trunk

Opening

To open the rear trunk, ensure Model 3 is in Park, then  do one of the following:

• Touch the associated Open button on the  touchscreen.

• Touch the rear trunk button on the mobile app.

• Press the switch located under the rear trunk's  exterior handle (a valid key must be detected).

CAUTION: Before opening the rear trunk in an  enclosed area (such as a garage), ensure the  opening height of the rear trunk is properly  adjusted to avoid low-hanging ceilings or objects  (see Adjusting Opening Height of Powered Trunk  on page 26).

Model 3 must be unlocked or detect a key before you  can use the switch to open the rear trunk.

When a door or trunk is open, the

touchscreen displays the Door Open

indicator light. The image of your Model 3

on the touchscreen also displays the open

trunk.

To open the rear trunk from inside the vehicle in the  unlikely situation that Model 3 has no power, see Interior  Emergency Trunk Release on page 27.

Adjusting Opening Height of  Powered Trunk

You can adjust the opening height of the powered trunk  (if equipped) to make it easier to reach or to avoid low hanging ceilings or objects (for example, a garage door  or light):

1. Open the trunk, then manually lower or raise it to the  desired opening height.

2. Press and hold the button on the underside of the  trunk for three seconds until you hear a confirmation chime.

3. Confirm that you have set it to the desired height by  closing the powered trunk, then reopening it.

CAUTION: Depending on configuration (such as  wheel selection), your vehicle's rear trunk can  open up to approximately 2 meters. Adjust the rear  trunk height to prevent it from coming into contact  with low ceilings or other objects.

Closing

• Touch the associated Close button on the  touchscreen.

• Press the switch located on the underside of the rear  trunk

WARNING: Before driving, ensure that the trunk is  securely latched in the fully-closed position by  lifting up on the bottom edge and confirming there is no movement.

Accessing the Cargo Area

To access the cargo area inside the rear trunk, pull up  the strap at the rear of the cargo cover. You can then  fold the cargo cover forward or remove it from Model 3.

Secure all cargo before moving Model 3, and place  heavy cargo in the lower trunk compartment.

26 MODEL 3 Owner's Manual

Rear Trunk Load Limits

Distribute the weight of cargo as evenly as possible  between the front and rear trunks.

CAUTION: Never load more than 40 kg in the  lower compartment of the rear trunk or more than  130 kg on the upper compartment. Doing so can  cause damage.

WARNING: When loading cargo, always consider  the vehicle's Technically Permissible Maximum  Laden Mass (TPMLM) (see Specifications on page  193). The TPMLM is the maximum allowable total  mass of the vehicle including all passengers, fluids, and cargo.

Interior Emergency Trunk Release

An illuminated mechanical release located inside the  rear trunk allows you to open the rear trunk from the  inside if Model 3 has no electrical power. This mechanical  release also allows a person locked inside to get out.

1. Firmly press and hold the illuminated button in the  direction of the arrow to release the latch.

2. While pressing the button, push the rear trunk open.

NOTE: The button glows for several hours after a brief  exposure to ambient light.

Rear Trunk

WARNING: Do not allow children to play inside the  trunk or become locked inside. An unrestrained  child could suffer serious injury or death in a crash.  A child could suffer heat exhaustion or death if  trapped in the vehicle, especially without climate  control on.

Storage Areas 27

Front Trunk

Opening

To open the front trunk, ensure Model 3 is in Park, and  then do one of the following before pulling the hood  open:

• Touch the associated Open icon on the touchscreen. • Touch the front trunk button in the mobile app.

When a door or trunk is open, the

touchscreen displays the Door Open

indicator light. The image of your Model 3

on the touchscreen also displays the open

front trunk.

WARNING: Before opening or closing the hood, it  is important to check that the area around the  hood is free of obstacles (people and objects).  Failure to do so may result in damage or serious  injury.

Closing

The Model 3 hood is not heavy enough to latch under its  own weight and applying pressure on the front edge or  center of the hood can cause damage.

To properly close the hood:

1. Lower the hood until the striker touches the latches.

2. Place both hands on the front of the hood in the  areas shown (in green), then press down firmly to  engage the latches.

3. Carefully try to lift the front edge of the hood to  ensure that it is fully closed.

CAUTION: To prevent damage:

• Apply pressure only to the green areas shown.  Applying pressure to the red areas can cause  damage.

• Do not close the hood with one hand. Doing so  applies concentrated force in one area and can  result in a dent or crease.

• Do not apply pressure to the front edge of the  hood. Doing so can crease the edge.

• Do not slam or drop the hood.

• To avoid scratches, don't have anything in your  hands (keys). Jewelry can also cause scratches.

WARNING: Before driving, you must ensure that  the hood is securely latched in the fully closed  position by carefully trying to lift the front edge of  the hood upward and confirming there is no  movement. It is the driver's responsibility to ensure  that the front trunk is properly closed before  driving.

If the front trunk is left open when you attempt to shift  out of Park, a notification requiring you to confirm your  intent to drive appears on the touchscreen. If you  choose to keep the front trunk open while driving, your  vehicle speed is limited.

The front trunk locks when:

28 MODEL 3 Owner's Manual

Front Trunk

• You lock Model 3 using the touchscreen, key or

mobile app.

• You leave Model 3 carrying your key (if Walk-Away

Door Lock on page 23 is turned on).

• Valet mode is active (see Valet Mode on page 84).

Front Trunk Load Limit

Distribute the weight of cargo as evenly as possible

between the front and rear trunks.

CAUTION: Never load more than 50 kg in the front

trunk. Doing so can cause damage.

WARNING: When loading cargo, always consider

the vehicle's Technically Permissible Maximum

Laden Mass (TPMLM) (see Specifications on page

193). The TPMLM is the maximum allowable total

mass of the vehicle including all passengers, fluids,

and cargo.

Interior Emergency Release

An illuminated interior release button inside the front

trunk allows a person locked inside to get out.

Press the interior release button to unlatch the front

trunk, then push up on the hood.

NOTE: The interior release button glows following a brief

exposure to ambient light.

WARNING: People should never climb inside the

front trunk. Never shut the front trunk when a

person is inside.

WARNING: Care should be taken to ensure that

objects inside the front trunk do not bump against

the release button, causing the hood to

accidentally open.

Storage Areas 29

Interior Storage

Center Console

In addition to housing an RFID transmitter that reads key  cards (see Keys on page 19), the center console includes  cup holders, two storage compartments, various  chargers (see Interior Electronics on page 11), and a rear  touchscreen.

To open the main storage compartment, squeeze the  latch under the front lip. Open the front storage  compartment by sliding its cover forward.

Rear Console

Your Model 3 has a rear console integrated in the center  of the second row seat back. Pull the console down to  access the rear cup holders or use it as an armrest.

NOTE: The rear console is self-locking in certain  situations and cannot be lowered. For example, while  driving on a steep slope or during hard accelerations.

Glovebox

To open the glovebox, touch Controls > Glovebox. The  glovebox automatically opens and the light turns on.

NOTE: On RHD (Right Hand Drive) vehicles, the  glovebox location illustrated above is arranged similarly,  but is mirrored on the left side of the vehicle.

To close the glovebox, push it upward until it latches into  the closed position.

For additional glovebox security, touch Controls >  Safety > Glovebox PIN to set a 4-digit PIN (see  Glovebox PIN on page 132).

NOTE: If you leave the glovebox open, its light  eventually turns off.

NOTE: The glovebox locks whenever closed and you  lock Model 3 using the mobile app, key card, you leave  Model 3 carrying your phone key (if Walk-Away Door  Lock is turned on), or if Valet mode is active (see Valet  Mode on page 84). It does not lock when Model 3 is  locked by touching the lock icon on the touchscreen.

WARNING: When driving, keep the glovebox  closed to prevent injury to a passenger if a  collision or sudden stop occurs.

Coat Hangers

Your Model 3 has a coat hanger on each side of the  vehicle above the rear window in the second row. Push  the coat hanger to release it. Push it again to retract it.

30 MODEL 3 Owner's Manual

Correct Driving Position

The seat, head support, seat belt and airbags work  together to maximize your safety. Using these correctly  ensures greater protection.

Position the seat so you can wear the seat belt correctly,  while being as far away from the front airbag as  possible:

1. Sit upright with both feet on the floor and the seat  back in an upright position.

2. Make sure you can easily reach the pedals and that  your arms are slightly bent when holding the steering  wheel. Your chest should be at least 25 cm from the  center of the airbag cover.

3. Place the shoulder section of the seat belt mid-way  between your neck and your shoulder. Fit the lap  section of the belt tightly across your hips, not across  your stomach.

Model 3 seats include integrated head supports that  cannot be adjusted or removed.

Front and Rear Seats Adjusting the Front Seats

1. Move seat forward/backward and adjust the seat’s  height and tilt angle up/down.

2. Adjust backrest.

3. Adjust lumbar support.

CAUTION: Do not move a front seat's backrest  fully forward when the seat is also in the fully  forward position. Doing so can cause the top of  the seat to hit, and potentially damage, the sun  visor.

WARNING: Before adjusting a front seat, check  that the area around the seat is free of obstacles  (people and objects).

Seating and Safety Restraints 31

Front and Rear Seats

WARNING: Do not adjust seats while driving.  Doing so increases the risk of a collision.

WARNING: Riding in a moving vehicle with the  seat back reclined can result in serious injuries in a  collision, as you could slide under the lap belt or  be propelled into the seat belt. Ensure your seat  back is reclined no more than 30 degrees when  the vehicle is moving.

Calibrating Seats

You can calibrate the driver seat. This is useful if you find your seat range limited or your driver profile does not  automatically adjust the seat for you. Navigate to  Controls > Service > Driver Seat, Steering & Mirrors  Calibration and follow the instructions on the  touchscreen.

WARNING: Ensure nothing is behind or

underneath the driver seat during calibration.  Failure to do so may cause serious injury.

Folding Rear Seats

Model 3 has a split rear seat that can fold forward.

NOTE: Driving with the rear seats folded forward can  increase the amount of perceived noise and/or vibration  coming from the rear of the vehicle (trunk, suspension,  etc.).

Before folding, remove items from the seats and the rear  footwell. To allow the rear seat backs to fold completely  flat, you may need to move the front seats forward.

To fold a rear seat, pull the corresponding lever and fold  the seat forward.

Raising Rear Seats

Before raising a rear seat, make sure that the seat belts  are not trapped behind the backrest.

Pull the seat back upward until it locks into place.

To confirm that the seat back is locked in the upright  position, try pulling it forward.

WARNING: Always ensure the seat backs are  locked in their upright position by pushing it  forward or rearward. Failure to do so increases the  risk of injury.

Head Supports

The front and second row seats include integrated head  supports that are not adjustable.

Seat Heaters

The front and rear seats operate at three levels from 3  (highest) to 1 (lowest). To operate the seat heaters, see  Operating Climate Controls on page 139.

WARNING: To avoid burns resulting from

prolonged use, individuals who have peripheral  neuropathy, or whose capacity to feel pain is  limited because of diabetes, age, neurological  injury, or some other condition, should exercise  caution when using the climate control system and  seat heaters.

32 MODEL 3 Owner's Manual

Seat Covers

WARNING: Do not use seat covers on a front seat.  Doing so could restrict deployment of the seat mounted side airbags if a collision occurs. Also, if  the vehicle is equipped with an occupant detection  system that is used to determine the status of the  passenger front airbag, seat covers may interfere  with this system.

Front and Rear Seats

Seating and Safety Restraints 33

Seat Belts

Wearing Seat Belts

Using seat belts and child safety seats is the most  effective way to protect occupants if a collision occurs.  Therefore, wearing a seat belt is required by law in most  jurisdictions.

All seats are equipped with three-point inertia reel seat  belts. Inertia reel belts are automatically tensioned to  allow occupants to move comfortably during normal  driving conditions.

The seat belt reel automatically locks to prevent  movement of occupants if Model 3 experiences a force  associated with hard acceleration, braking, cornering, or  an impact in a collision.

Seat Belt Reminders

The seat belt reminder on the touchscreen

alerts you if a seat belt for an occupied

driver or passenger seat is unbuckled. If all

occupants are buckled up and the

reminder stays on, re-buckle seat belts to

ensure they are correctly latched. Also

remove any heavy objects (such as a

briefcase) from an unoccupied seat. If the

reminder light continues to stay on,

contact Tesla.

WARNING: Seat belts must be worn by passengers  in all seating positions.

NOTE: In regions where regulations require seat belt  reminders in rear seating positions, these reminders  cannot be disabled. To cancel the reminder in an  unoccupied seat when an object is detected, you must  either fasten the seat belt or remove the object.

To Fasten a Belt

1. Ensure correct positioning of the seat. See (see  Correct Driving Position on page 31) for details on the  correct position of the driver's seat.

2. Draw the belt out smoothly, ensuring the belt lays flat across the pelvis, chest and mid-point of your collar  bone, between the neck and shoulder. Ensure the belt  is routed correctly and is not twisted. Never sit on the  seat belt or any seat belt component.

WARNING: A twisted or incorrectly routed seat  belt can cause damage and interfere with the  functionality of the seat belt system.

3. Insert the latch plate into the buckle and press  together until you hear a click indicating it is locked  in place.

4. Pull the belt to check that it is securely fastened.

5. Pull the diagonal part of the belt toward the reel to  remove excess slack.

To Adjust the Shoulder Anchor  Height

Model 3 is equipped with an adjustable shoulder anchor  for each front seat to ensure that the seat belt is  positioned correctly. The seat belt should lay flat across  the mid-point of your collar bone while in the correct  driving position (see Correct Driving Position on page  31). Adjust the height of the shoulder anchor if the seat  belt is not positioned correctly:

1. Press and hold the button on the shoulder anchor to  release the locking mechanism.

2. While holding the button, move the shoulder anchor  up or down, as necessary, to correctly position the  seat belt.

3. Release the button on the shoulder anchor so that it  locks into position.

4. Without pressing the button, pull on the seat belt  webbing and attempt to move the shoulder anchor  downward to check that it is locked into position.

34 MODEL 3 Owner's Manual

WARNING: Ensure that the seat belt is positioned  correctly and that the shoulder anchor is locked  into position before driving. Riding in a moving  vehicle with the seat belt positioned incorrectly or  with the shoulder anchor not locked into position  can reduce the effectiveness of the seat belt in a  collision.

To Release a Belt

Hold the belt near the buckle to prevent the belt from  retracting too quickly, then press the button on the  buckle. The belt retracts automatically. Ensure there is  no obstruction that prevents the belt from fully  retracting. The belt should not hang loose. If a seat belt  does not fully retract, contact Tesla.

Wearing Seat Belts When Pregnant

Do not put the lap or shoulder sections of the seat belt  over the abdominal area. Wear the lap section of the  belt as low as possible across the hips, not the waist.  Position the shoulder portion of the belt between the  breasts and to the side of the abdomen. Consult your  doctor for specific guidance.

WARNING: If the seat belt is uncomfortable, adjust  the seating position instead of wearing the seat  belt incorrectly.

WARNING: Never place anything between you and  the seat belt to cushion the impact in the event of  a collision.

Seat Belts

Seat Belt Pre-tensioners

The front seat belts are equipped with pre‑tensioners  that work in conjunction with the airbags in a collision.  The pre‑tensioners automatically retract both the seat  belt lower anchor and the upper shoulder webbing,  reducing slack in both the lap and diagonal portions of  the belts, resulting in reduced forward movement of the  occupant.

If the pre-tensioners and airbags did not activate in an  impact, this does not mean they malfunctioned. It  usually means that the strength or type of force needed  to activate them was not present.

The rear outboard seats are equipped with shoulder pre tensioners to retract the seat belt webbing to reduce  forward movement of the occupant.

WARNING: Do not bend, sit on, or interfere with  the pre-tensioner assembly. Doing so can cause  damage that interferes with the proper

functionality of the seat belt system.

WARNING: Once the seat belt pre-tensioners have  been activated, they must be replaced. After any  collision, have the airbags, seat belt pre-tensioners  and any associated components checked and, if  necessary, replaced.

Testing Seat Belts

To confirm that seat belts are operating correctly,  perform these checks on each seat belt.

1. With the seat belt fastened, give the webbing nearest  the buckle a quick and forceful pull. The buckle  should remain securely locked.

Seating and Safety Restraints 35

Seat Belts

2. With the seat belt fastened, give the webbing closest  to the door a quick and forceful pull. The permanent  seat belt attachment should remain securely locked.  Never attempt to remove this attachment.

3. With the belt unfastened, unreel the webbing to its  limit. Check that unreeling is free from snags, and  visually check the webbing for wear. Allow the  webbing to retract, checking that retraction is  smooth and complete.

4. With the webbing half unreeled, hold the tongue  plate and pull forward quickly. The mechanism should  lock automatically and prevent further unreeling.

If a seat belt fails any of these tests, repair immediately.  Do not allow occupants to sit in a seat with a failed seat  belt.

For information about cleaning seat belts, see Seat Belts  on page 184.

Seat Belt Warnings

WARNING: Seat belts should be worn by all  occupants at all times, even if driving for a very  short distance. Failure to do so increases the risk  of injury or death if a collision occurs.

WARNING: Secure small children in a suitable child  safety seat as described in the Owner's Manual.  Always follow the child safety seat manufacturer's  instructions when installing.

WARNING: Ensure that all seat belts are worn  correctly. An improperly worn seat belt increases  the risk of injury or death if a collision occurs.

WARNING: Never sit on top of any seat belt  component. Doing so can cause damage or  improper deployment of safety equipment.

WARNING: Do not wear seat belts over hard,  fragile or sharp items in clothing, such as pens,  keys, eyeglasses, etc. The pressure from the seat  belt on such items can cause injury.

WARNING: Seat belts should not be worn with any  part of the strap twisted.

WARNING: Each seat belt assembly must be used  by one occupant only. It is dangerous to put a seat  belt around a child being carried on an occupant’s  lap.

WARNING: Seat belts that have been worn in a  collision must be inspected or replaced by Tesla,  even if damage to the assembly is not obvious.

WARNING: Seat belts that show signs of wear, or  have been cut or damaged in any way must be  replaced immediately.

WARNING: Avoid contaminating a seat belt’s  components with any chemicals, liquids, grit, dirt  or cleaning products. If a seat belt fails to retract  or latch into the buckle, it must be replaced  immediately. Use the mobile app to schedule a  service appointment.

WARNING: Do not make modifications or  additions that can prevent a seat belt mechanism  from taking up slack, or that can prevent a seat  belt from being adjusted to remove slack. A seat  belt with slack greatly reduces occupant  protection.

WARNING: Do not make modifications that can  interfere with the operation of a seat belt, or that  can cause a seat belt to become inoperable.

WARNING: Do not use after market comfort and  convenience products that attach to the seat belts.

WARNING: When seat belts are not in use, they  should be fully retracted and not hanging loose. If  a seat belt does not fully retract, schedule a  service appointment.

WARNING: The seat belt system has no user  serviceable parts and may contain pyrotechnics.  Do not disassemble, remove, or replace  components.

36 MODEL 3 Owner's Manual

Seat Children in Rear Seats Only

Child Safety Seats

WARNING: The front passenger seat has an airbag that is always ON and may inflate when a collision occurs.  Never seat a child on the front passenger seat, even if using a child restraint system or booster seat. DEATH or  SERIOUS INJURY could occur.

Refer to the following label fitted to the sun visor.

The Passenger Airbag On indicator displays in the top corner of the touchscreen to indicate that the  passenger front airbag may inflate if a collision occurs.

Suitability and Fitting of Child Restraint Systems

All Model 3 seat belts are designed for adults. When seating infants and children, you must:

• Use a child restraint system appropriate for the child’s age, weight, or size (see Recommended Child Restraint  Systems on page 38).

• Use a child restraint system that is appropriate for the specific seating position in Model 3. Each passenger seat in  Model 3 supports a broad range of options. For details on what type of child restraint system can be used in each  passenger seat, see Front Passenger Seat on page 40, and Rear Seats on page 41.

• Properly install the child restraint system by following the manufacturer's instructions (see Installing Belt-based  Child Restraint Systems on page 42 and Installing ISOFIX Child Restraint Systems on page 42 for general  guidelines).

Storage Areas 37

Child Safety Seats

Recommended Child Restraint Systems

Although all weight groups can occupy any passenger seat in Model 3, the type of child restraint system that can be  used in each seat can vary. For example, only belt-based child restraint systems (as defined in Two Installation  Methods on page 39) can be used in the rear center seat.

Choose a child restraint system that meets AS/NZS 1754. For more information about child seats, go to  www.childcarseats.com.au. Child restraint systems purchased outside of Australia may not meet these standards.

WARNING: To ensure safety of infants and children, it is advisable to purchase a new child restraint system.  However, if a used child restraint system is necessary, heed these warnings:

• Do not use a child restraint system that is more than 10 years old.

• Ensure the child restraint system meets AS/NZS 1754. This information should be visible on a label affixed to  the child restraint system.

• Confirm that the child restraint system was not being used in a vehicle that was involved in a collision.

• Ensure that all components (buckles, etc.) of the child restraint system operate smoothly and there are no  signs of wear or damage (splits, cracks etc.).

• Ensure the harnesses on the child restraint system are not frayed, worn or damaged

• Ensure the child restraint system has an instruction manual.

38 MODEL 3 Owner's Manual

Child Safety Seats

Seating Larger Children

If a child is too large to fit into a child restraint system, but too small to be safely secured using the vehicle's seat  belts, use a booster seat appropriate for the child’s age and size. When using and installing a booster seat, carefully  follow the instructions provided by the manufacturer.

Two Installation Methods

NOTE: Always install child restraint systems by following the instructions provided by the manufacturer of the child  restraint system.

Among many other variants, there are two general types of child restraint systems based on how they are secured in  the seat:

• Belt-based - secured using the vehicle’s seat belts (see Installing Belt-based Child Restraint Systems on page 42).

• ISOFIX - secured to anchor bars built into the vehicle's seats (see Installing ISOFIX Child Restraint Systems on  page 42).

Some child restraint systems can be installed using either method. Refer to the instructions provided by the  manufacturer of the child restraint system for detailed installation instructions.

In Model 3, belt-based child restraints can be installed in any rear passenger seat and ISOFIX systems can be  installed in either of the rear outboard seats. Specific details about the type of child restraint system that can be  used in each seat is provided next.

NOTE: ISOFIX is an international standard for integrated anchors used in passenger vehicles to attach child safety  seats.

WARNING: Do not use ISOFIX anchors with child restraint system or booster seats that have an integral safety  belt where the combined weight of the child plus the child restraint system exceeds 33 kg.

Seating and Safety Restraints 39

Child Safety Seats

Front Passenger Seat

WARNING: Never seat a child in a rear-facing child restraint system on the front passenger seat. Doing so can  cause serious injury or death.

NOTE: The front passenger seat is not equipped with lower anchor bars to support the installation of ISOFIX child  restraint systems. The front passenger seat is also not equipped with an anchor for upper tether straps.

WARNING: A passenger less than four years old must not be seated in the front row of a vehicle that has two  or more rows of seats.

WARNING: Children between the ages from four to seven can not travel in the front seat of a vehicle that has  two or more rows, unless all rear seats are occupied by younger children seated in approved child restraint  systems. In this situation, the oldest or largest child can be seated in the front passenger seat using a booster  seat and the seat belt.

WARNING: Children in booster seats must be restrained by a suitable and approved lap and sash type seat belt  that is properly adjusted and fastened, or by a suitable and approved child safety harness that is properly  adjusted and fastened.

40 MODEL 3 Owner's Manual

Child Safety Seats

Rear Seats

Infants and children can occupy the rear outboard seat using rear-facing belt-based or ISOFIX child restraint  systems. Children can use forward-facing child restraint systems when appropriate for their weight, size, and age.

NOTE: To accommodate large rear-facing child restraint systems, you may need to move the corresponding front  seat forward, raise the seat upward (2 cm from its lowest position), and angle the seat back as needed.

NOTE: Child safety seats installed in a rear outboard seating position require the use of upper tether straps (see  Attaching Upper Tether Straps on page 43).

Larger children can also occupy a rear outboard seat using a booster seat, properly installed as described in the  instructions provided by the child restraint system manufacturer.

Model 3 is compatible with child restraint systems Type A, A2, A4, B, D, E, and F.

WARNING: Do not use Easy Entry (as described in Driver Profiles on page 83) to automatically move the  driver's seat to the full rearward position if a child safety seat is installed on a rear seat behind the driver's seat.  With reduced clearance, the movement of the seat may impact a child's legs, cause injury, or dislodge the seat.

Seating and Safety Restraints 41

Child Safety Seats

Installing Belt-based Child Restraint

Systems

Always follow the detailed instructions provided by the

manufacturer of the child restraint system. Follow these

general guidelines for belt-based child restraint systems:

• Ensure that the child restraint system is appropriate

for the weight, height, and age of the child.

• Avoid dressing the child in bulky clothing.

• Do not place any objects between the child and the

child restraint system.

• Adjust the child restraint system's harnesses for every

child, every trip.

1. Place the child restraint system in the appropriate

seat and fully extend the seat belt. Route and buckle

the seat belt in accordance with the instructions

provided by the manufacturer of the child restraint

system.

2. Allow the seat belt to retract, and remove all slack in  the seat belt while firmly pushing the child restraint  system into the Model 3 seat.

3. Attach the upper tether strap(s), as required by the  manufacturer of the child restraint system (see  Attaching Upper Tether Straps on page 43).

Installing ISOFIX Child Restraint  Systems

Model 3's rear outboard seating positions are equipped  with ISOFIX anchor bars. These anchor bars are located  between the seat's cushion and back rest. The exact  location of each anchor bar is identified by a marking  (illustrated below) located on the seat back, directly  above its associated anchor bars.

To install an ISOFIX child restraint system, carefully read  and follow the instructions provided by the  manufacturer of the child restraint system. These  instructions describe how to slide the child restraint  system onto the seat's anchor bars until you hear it  "click" into place. You may need to push the child  restraint system firmly against the seat back to ensure it  fits snugly.

42 MODEL 3 Owner's Manual

Adjust until the child restraint system is fitted firmly against the seat back. Ensure the child restraint system  fits snugly.

Before seating a child, ensure that the child restraint  system is securely installed. Grasp the front of the child  restraint system with one hand on each side, and  attempt to:

• Twist the child restraint system from side to side. • Pull the child restraint system away from the seat.

Child Safety Seats

If the child restraint system moves away from the seat,  both latches are not fully engaged onto the seat's  anchor bars. You must reinstall it and try again. It is  critical that both latches on the child restraint system  are fully engaged.

WARNING: Do not use ISOFIX anchors with child  restraint system or booster seats that have an  integral safety belt where the combined weight of  the child plus the child restraint system exceeds 33  kg.

Attaching Upper Tether Straps

Model 3's rear seats support the use of upper tether  straps. Attach its hook to the anchor point located  behind the associated seat.

NOTE: The rear headrests do not move.

NOTE: Tighten upper tether straps according to the  instructions provided by the manufacturer of the child  restraint system.

The anchor points for tether straps are located on the  shelf behind the rear seats.

To access an anchor point, press down on the back of its  cover.

Seating and Safety Restraints 43

Child Safety Seats

For dual-strap tethers, position a strap on each side of  the head support.

For single-strap tethers at the outboard seating  positions, route the strap over the outside-facing side of  the head support (same side of the head support as the  seat belt retraction mechanism).

For a single strap tether in the rear center seating  position, route the strap over the top center of the head  support.

For dual strap tether in the rear center seating position,  route the straps around each side of the headrest.

44 MODEL 3 Owner's Manual

Child Restraint System Warnings

WARNING: Never seat a child in a rear-facing child  restraint system on the front passenger seat when  the passenger front airbag is active. Doing so can  cause serious injury or death.

WARNING: Some child restraint systems are  designed to be secured in vehicle seats by lap  belts or the lap belt portion of a lap-shoulder belt.  Children could be endangered in a crash if their  child restraint systems are not properly secured in  the vehicle.

WARNING:

A passenger less than six months old must be  restrained in a suitable and properly fastened and  adjusted rear-facing approved child restraint  system. A passenger six months or older, but less  than four years old, must be restrained in a  suitable and properly fastened and adjusted:

• rear-facing approved child restraint system; or,

• forward-facing approved child restraint system  equipped with a built-in harness.

Infants and small children should ride in a rear facing child restraint system as long as possible,  until they exceed the maximum weight or height  permitted by rear-facing child restraint systems.

WARNING: Do not allow a baby or infant to be  held on an adult's lap. All children should be  restrained in an appropriate child restraint system  at all times.

WARNING: To ensure children are safely seated,  follow all instructions provided in this document  and by the manufacturer of the child restraint  system.

WARNING: Children should ride in a rear-facing  child restraint system using the seat’s integrated 5- point harness for as long as possible until they  reach the maximum size or weight limit of the  rear-facing child restraint system.

WARNING: When seating larger children, make  sure the child's head is supported and the child’s  seat belt is properly adjusted and fastened. The  shoulder portion of the belt must be away from  the face and neck, and the lap portion must not be  over the stomach.

WARNING: Never attach two child restraint  systems to one anchor point. In a collision, one  anchor point may be incapable of securing both  seats.

WARNING: Child restraint anchor bars are

designed to withstand only those loads imposed  by correctly fitted child restraints. Under no  circumstances are they to be used for adult seat  belts, harnesses, or for attaching other items or  equipment to the vehicle.

Child Safety Seats

WARNING: Always check harnesses and tether  straps for damage and wear.

WARNING: Never leave a child unattended, even if  the child is secured in a child restraint system.

WARNING: Never use a child restraint system that  has been involved in a collision. Discard the seat  and have it replaced as described in the child  restraint system manufacturer’s instructions.

Seating and Safety Restraints 45

Airbags

Location of Airbags

Airbags are located in the approximate areas shown below. Airbag warning information is printed on the sun visors.

Model 3 is equipped with an airbag and lap/shoulder belt at both front seating positions. The airbag is a  supplemental restraint at those seating positions. All occupants, including the driver, should always wear their seat  belts whether or not an airbag is also provided at their seating position to minimize the risk of severe injury or death  in the event of a crash.

NOTE: On RHD (Right Hand Drive) vehicles, the locations of the passenger and driver airbags are reversed.

1. Front airbags

2. Seat-mounted side airbags

NOTE: Your vehicle is also equipped with a seat-mounted airbag on the inside portion of the driver's seat. 3. Curtain airbags

46 MODEL 3 Owner's Manual

How Airbags Work

Airbags

Types of Airbags

Airbags inflate when sensors detect an impact that  exceeds deployment thresholds. These thresholds are  designed to predict the severity of a crash in time for  the airbags to help protect the vehicle's occupants.  Airbags inflate instantly with considerable force  accompanied by a loud noise. The inflated bag, together  with the seat belts, limits movement of occupants to  reduce the risk of injury.

Front airbags are not ordinarily designed to inflate in  rear collisions, rollovers, side collisions and when braking  heavily or driving over bumps and potholes. Likewise,  front airbags may not inflate in all frontal collisions, such  as minor front collisions, underride collisions, or minor  impacts with narrow objects (such as posts or poles).  Significant superficial damage can occur to the vehicle  without the airbags inflating and, conversely, a relatively  small amount of structural damage can cause airbags to  inflate. Therefore, the external appearance of the vehicle  after a collision does not represent whether or not the  front airbags should have inflated.

WARNING: Before modifying your vehicle to  accommodate a person with disabilities in a way  that may affect the airbag system, use the mobile  app to schedule a service appointment.

Passenger Front Airbag

Model 3 has the following types of airbags:

• Front airbags: The front airbags are designed to  reduce injuries if larger children or adults are riding in  the front seats. Follow all warnings and instructions  related to seating a child on the front passenger seat  (if permitted in your market region). See Child Safety  Seats on page 37.

• Seat-mounted side airbags: A seat-mounted side  airbag in the front seats helps protect the pelvis and  the thorax region of the torso; the seat-mounted far  side airbag in the inside portion of the driver’s seat  helps protect the head and torso. The seat-mounted  side airbags on both the impacted and non-impacted  side of the vehicle will inflate in the event of severe  side impact or severe offset frontal impact.

• Curtain airbags: Curtain airbags help protect the  head. Curtain airbags on both the impacted and non impacted side of the vehicle will inflate only if a  severe side impact occurs, or if the vehicle rolls over.

Model 3 has a passenger front airbag that is always ON. Children must ride in the rear seats only.

The Passenger Airbag On indicator displays in the top corner of the touchscreen to indicate that the  passenger front airbag may inflate if a collision occurs.

WARNING: The front passenger seat has an airbag that is always ON. Never seat a child in the front passenger  seat, even if using a child restraint system or booster seat. DEATH or SERIOUS INJURY can occur.

Inflation Effects

WARNING: When airbags inflate, a fine powder is  released. This powder can irritate the skin and  should be thoroughly flushed from the eyes and  from any cuts or abrasions.

After inflation, the airbags deflate to provide a gradual  cushioning effect for the occupants and to ensure the  driver’s forward vision is not obscured.

If airbags have inflated, or if your vehicle has been in a  collision, your vehicle requires servicing before it will  power up. In addition, your airbags, seat belt pre tensioners and any associated components must be  checked, and if necessary, replaced. Use the mobile app  to schedule a service appointment immediately.

In a collision, in addition to the airbags inflating: • Doors unlock.

• Hazard warning lights turn on.

• Interior lights turn on.

• High voltage is disabled.

• Windows go to the vent position.

• Vehicle applies the brakes to come to a stop.

NOTE: Depending on the nature of the impact and the  forces involved, doors may not unlock in a collision  and/or damage may prevent them from opening. In such  cases, the door may need to be opened using the  interior manual release, or other means of extrication  (for example, exiting through another door, breaking the  window, etc.).

NOTE: In some collisions, even if airbags did not inflate, high voltage may be disabled and you will be unable to  power up and drive. Use the mobile app to schedule a  service appointment immediately.

Seating and Safety Restraints 47

Airbags

Airbag Warning Indicator

The airbag indicator on the touchscreen

remains lit if the airbag system is

malfunctioning. The only time this indicator  should light up is briefly when Model 3 first

powers up, in which case it turns off within  a few seconds. If it remains lit, use the

mobile app to schedule a service

appointment immediately and do not drive. Airbag Warnings

WARNING: All occupants, including the driver,  should always wear their seat belts, whether or not  an airbag is also provided at their seating position,  to minimize the risk of severe injury or death in the  event of a collision.

WARNING: Front seat occupants should not place  their arms over the airbag module, as an inflating airbag can cause fractures or other injuries.

WARNING: Do not use seat covers on Model 3.  Doing so could restrict deployment of the seat mounted side airbags if a collision occurs. It can  also reduce the accuracy of the Occupant

Classification System (OCS), if equipped.

WARNING: Airbags inflate with considerable  speed and force, which can cause injury. To limit  injuries, ensure that occupants are wearing seat  belts and are correctly seated, with the seat  positioned as far back as possible.

WARNING: Children should not be seated on the  front passenger seat unless permitted by

regulations in your market region. Follow all  regulations in your region for the appropriate way  to seat a child based on the child's weight, size,  and age. The safest place to seat infants and  young children is in a rear seating position. Seating  an infant or child in a rear-facing child restraint  system on a seat equipped with an operational  airbag can cause serious injury or death.

WARNING: Do not use a rear-facing child restraint  system on a seat with an operational airbag in  front of it. Doing so can cause injury or death if the  airbag inflates.

WARNING: To ensure correct inflation of the side  airbags, maintain an unobstructed gap between an  occupant’s torso and the side of Model 3.

WARNING: Passengers shouldn't lean their heads  against doors or windows. Doing so can cause  injury if a curtain airbag inflates.

WARNING: Do not allow passengers to obstruct  the operation of an airbag by placing feet, knees  or any other part of the body on or near an airbag.

WARNING: Do not attach or place objects on or  near the front airbags, the side of the front seats,  the headliner at the side of the vehicle, or any  other airbag cover that could interfere with  inflation of an airbag. These include but are not  limited to: steering wheel covers, decals, seat  cushions, pillows, etc. Objects can cause serious  injury if the vehicle is in a collision severe enough  to cause the airbag to inflate.

WARNING: Following inflation, some airbag  components are hot. Do not touch until they have  cooled.

48 MODEL 3 Owner's Manual

The Tesla mobile app allows you to communicate with  Model 3 remotely using your iPhone® or Android™  phone.

NOTE: The information below may not represent an  exhaustive list of the functions available on the Tesla  mobile app. To ensure access to new and improved  features, download updated versions of the mobile app  as they become available.

To Use the Mobile App

To set up the Tesla mobile app to communicate with  your Model 3:

1. Download the Tesla mobile app to your phone.

2. Log in to the Tesla mobile app by entering your Tesla  account credentials.

3. Enable mobile access to your Model 3 by touching  Controls > Safety > Allow Mobile Access.

4. Turn your phone's Bluetooth setting ON and ensure  that Bluetooth is turned on within your phone's  global settings for the Tesla mobile app. For example,  on your phone, navigate to Settings, choose the Tesla  mobile app, and ensure the Bluetooth setting is  enabled.

Your phone and vehicle must both be actively  connected to cellular service or Wi-Fi for the mobile app  to communicate with your vehicle. Tesla recommends  that you always have a functional physical key readily  available if parking in an area with limited or absent  cellular service, such as an indoor parking garage.

NOTE: In the event that you require lockout assistance  from Tesla due to a non-warranty issue, such as having  limited cellular connectivity and having no secondary  key available, your expenses are not covered under the  Roadside Assistance policy.

NOTE: Tesla does not support the use of third party  applications to contact Model 3.

Overview

When both your phone and the vehicle have internet  service, the Tesla mobile app's home screen allows you  to:

• Lock or unlock your vehicle.

• Enable or disable the heating or air conditioning and  monitor the cabin climate.

• Check your vehicle's charging information. Charging  details also appear when a charging cable is plugged  in.

• Open or close the charge port.

NOTE: Twisting red lines next to the Battery icon  indicate that the Battery is actively heating up  (including while charging or preparing to charge).

Mobile App

• See where your vehicle is located.

• View your vehicle’s estimated range.

• Open the front trunk.

• View your vehicle’s odometer, VIN, and current  software version.

Media settings appear on the mobile app to pause, play,  rewind, fast forward, and adjust the volume of the media  currently playing in the vehicle. You may need to enable

Media settings by touching Audio Settings > Options >  Allow Mobile Control.

For supported video sources, send videos to Tesla  Theater by sharing the link through the mobile app.  Navigate to the movie, show, or video you want to play  on your phone and touch the share button. Share the  video with the Tesla app and it appears on the  touchscreen if Model 3 is in Park.

Profile

In the Profile tab located at the top corner, you can:

• Switch to a different vehicle associated with your  Tesla account, if you have access to more than one.

• Navigate the Tesla Shop.

• Manage your account information and view your  order history.

• View and customize notifications you receive under  the Settings tab, such as Calendar sync, when your  security alarm has been triggered, charging updates,  and new software updates. You can start updates  from afar and check its progress.

Controls

The Controls tab allows you to do the following:

• Open the front or rear trunk.

• Lock or unlock Model 3 from afar.

NOTE: Your vehicle does not automatically re-lock if  you unlock from the mobile app.

• Open or close the charge port.

• Flash the lights or honk the horn to find where Model  3 is parked.

• Enable Keyless Driving.

NOTE: Keyless Driving can be used when you do not  have your key or to bypass PIN to Drive in cases  where you forgot your PIN or your touchscreen is  unresponsive (see PIN to Drive on page 132).

• Open and close your garage door if your vehicle has  a programmed HomeLink connection, if available (see  Smart Garage on page 57).

• Vent the windows.

Connectivity 49

Mobile App

Climate

You can check the interior temperature and heat or cool  the cabin before driving (even if it’s in a garage), control  the seat heaters, and defrost the windshield:

• Enable or disable Defrost Car, which helps melt snow,  ice, and frost on the windshield, windows, and  mirrors, by swiping up from the bottom of the screen.

• Enable or disable Dog Mode or Camp Mode.

• Enable Cabin Overheat Protection, which prevents  the cabin from getting too warm in hot ambient  conditions. You can choose whether you want the  A/C or just the fan to run when the temperature in  the cabin exceeds 40° C or the selected temperature  (if available). See Operating Climate Controls on page  139 for more information.

• Vent or close the windows.

•

Using the mobile app to precondition Model 3 also  warms the Battery as needed. The mobile app will notify  you once your vehicle has reached the desired  preconditioning temperature.

NOTE: In some vehicles, depending on vehicle  specifications and date of manufacture, using the mobile  app to defrost Model 3 also thaws ice on the charge port  latch. This is useful in extremely cold weather or icy  conditions in which the charge port latch can freeze in  place, preventing you from removing or inserting the  charge cable.

Location

Locate Model 3 with directions, or track its movement  across a map.

Summon

You can park or retrieve Model 3 using Summon (see  Summon on page 114) or Smart Summon (see Smart  Summon on page 116).

Schedule

Enable scheduled charging or departure, and  precondition the vehicle. See #unique_183 on page for  more information. Scheduled charging or departure can  also be saved based on a preferred location.

Security

The Security tab allows you to do the following:

• Pair your phone to the vehicle (see Phone Key on  page 19).

• Enable or disable Sentry Mode (see How to Use  Sentry Mode (Camera + App) on page 136).

• Enable or disable Valet Mode (see Valet Mode on  page 84).

• Enable or disable Speed Limit Mode and receive  notifications when the vehicle's driving speed is  within approximately 5 km/h of your selected  maximum speed (see Speed Limit Mode on page  132).

Upgrades

View and purchase the latest upgrades available for your  vehicle, such as full self-driving.

Service

See Schedule Service on page 174 for information on  how to schedule service through the mobile app.

Roadside

View roadside resources and request roadside  assistance (where applicable). For more information on  Roadside Assistance, see Contacting Tesla Roadside  Assistance on page 206.

Granting Access to a Second Driver

Add and remove access permission for an additional  driver from the Tesla mobile app.

NOTE: Tesla mobile app version 4.3.1 or higher is  required. Additional drivers can either use a previously  registered Tesla Account or use the app to create a new  Tesla Account.

To add an additional driver, in the Tesla mobile app from  the vehicle home screen, go to Security > Add Driver  and follow the onscreen instructions.

NOTE: The additional driver has access to all app  features except purchasing upgrades.

To remove access, use the mobile app and go to  Security > Manage Drivers and follow the onscreen  instructions.

50 MODEL 3 Owner's Manual

Wi-Fi

Wi-Fi is available as a data connection method and is

often faster than cellular data networks. Connecting to

Wi-Fi is especially useful in areas with limited or no

cellular connectivity. To ensure fast, reliable delivery of

software and map updates, Tesla recommends leaving

Model 3 connected to a Wi-Fi network whenever

possible (for example, when parked at home).

To connect to a Wi-Fi network:

1. Touch Controls > Wi-Fi icon at the top of the

Controls screen. Model 3 begins to scan and display

detected Wi-Fi networks that are within range.

When connected to Wi-Fi, the Wi-Fi icon displays at

the top of the touchscreen.

NOTE: If a known Wi-Fi network does not appear in

the list, move Model 3 closer to the access point or

consider using a range extender.

NOTE: When connecting to a 5GHz network (if

available), check which channels are supported in

your region.

5GHz Network Channels Supported

| 36-48 | 52-64 | 100-140 | 149-165 |
| --- | --- | --- | --- |
|  |  | ✓ | ✓ |

2. Select the Wi-Fi network you want to use, enter the

password (if necessary), then touch Confirm.

NOTE: Model 3 does not currently support

connections to captive Wi-Fi networks (a captive Wi

Fi, commonly used by public hotspots, requires you

to access a custom web portal and agree to terms of

service prior to allowing you to log in).

3. Model 3 connects to the Wi-Fi network. Whenever

the network is within range, Model 3 connects to it

automatically.

You can also connect to a hidden network that isn't

shown on the list of scanned networks. Just touch Wi-Fi

Settings, enter the name of the network in the resulting

dialog box, select the security setting, then touch Add

Network.

NOTE: If more than one previously connected network is

within range, Model 3 connects to the one most recently

used.

NOTE: At Tesla Service Centers, Model 3 automatically

connects to the Tesla Service Wi-Fi network.

Hotspots and Connectivity

You can also use a mobile hotspot or your phone's

Internet connection via Wi-Fi tethering (subject to fees

and restrictions of your mobile carrier) to access the

internet. To remain connected to Wi-Fi when shifting

into Drive or Reverse, choose the connection from the

Wi-Fi settings screen, then touch Remain connected in

Drive.

Connectivity 51

Bluetooth

Bluetooth® Compatibility

You can use various Bluetooth devices in

Model 3 provided it is paired and within

operating range. For example, you can pair  your Bluetooth-capable phone so you can

use it hands-free. In addition to phones,

you can pair other Bluetooth-enabled

devices with Model 3. For example, you can  pair an iPod Touch, iPad, Android tablet,

etc. from which you can play music.

Before using your phone or other Bluetooth device with  Model 3, you must pair it. Pairing sets up Model 3 to  communicate with supported Bluetooth-capable  devices. You can pair up to ten Bluetooth phones. Unless  you've specified a specific phone as a Priority Device, or  if the phone specified as Priority Device is not within  range, Model 3 always connects to the last phone that  was used (provided it is within range). To connect to a  different phone, see Connecting to a Paired Device on  page 53.

NOTE: Authenticating your phone to use as a key (see  Keys on page 19) does not also allow you to use the  phone hands-free, play media from it, etc. You must also  pair it as described below.

NOTE: On many phones, Bluetooth turns off if the  phone’s battery is low.

NOTE: Although Bluetooth typically supports wireless  communication over distances of up to approximately  nine meters, performance can vary based on the phone,  or other device, you are using.

Pairing a Bluetooth Device

Pairing allows you to use your Bluetooth-capable phone  hands-free to make and receive phone calls, access your  contact list, recent calls, etc. It also allows you to play  media files from your phone. Once a phone is paired,  Model 3 can connect to it whenever the phone is within  range.

To pair a phone, follow these steps while sitting inside  Model 3:

1. Ensure both the touchscreen and the phone are  powered on.

2. On your phone, enable Bluetooth and ensure it is  discoverable.

NOTE: On some phones, this may require you to go  to Bluetooth Settings for the remainder of the  procedure.

3. Touch the Bluetooth icon located at the top of the  Controls screen.

4. On the touchscreen, touch Add New Device > Start  Search. The Bluetooth settings screen displays a list  of all available Bluetooth devices within operating  distance.

5. Choose the phone (or device) with which you want to  pair. When pairing a phone, the Bluetooth settings  screen displays a randomly generated number. Your  phone also display a number.

6. Check that the number displayed on your phone  matches the number on the Bluetooths settings  screen. Then, on your phone, confirm that you want  to pair.

7. If prompted on your phone, specify whether you  want to allow Model 3 to access your personal  information, such as calendar, contacts and media  files (see Importing Contacts and Recent Calls on  page 52).

If you are experiencing issues importing or connecting  to Bluetooth, see Troubleshooting Bluetooth on page 53  for more information.

When paired, Model 3 automatically connects, and the  Bluetooth settings screen displays the Bluetooth symbol  next to the device name to indicate that the connection  is active.

You can display the Bluetooth settings screen at any  time and change the settings associated with a  connected device. For example, you can designate a  connected phone as the Priority Device. This is useful in  situations where you have connected more than one  phone, and both phones are frequently used in Model 3  at the same time. Model 3 automatically attempts to  connect to the priority device before others. If a priority  device is not specified, or is not within range, Model 3  connects to the most recently used phone (if  applicable).

Importing Contacts and Recent  Calls

Once a phone is paired, use the Bluetooth settings  screen to specify whether you want to allow access to  your phone’s contacts, recent calls and text messages. If  you allow access, you can use the phone app to make  calls and send messages to people in your list of  contacts and on your recent calls list (see Phone,  Calendar, and Web Conferencing on page 55). Before  contacts can be imported, you may need to either set  your phone to allow syncing, or respond to a popup on  your phone to confirm that you want to sync contacts.  This varies depending on the type of phone you are  using. For details, refer to the documentation provided  with your phone.

If you are having trouble importing contacts or paiting  with Bluetooth, see Troubleshooting Bluetooth on page  53 for more information.

52 MODEL 3 Owner's Manual

NOTE: You can turn access to your contacts and recent  calls on or off at any time by touching the Bluetooth  icon on the touchscreen, choosing your phone, and then  changing the associated access settings.

Unpairing a Bluetooth Device

If you want to disconnect your phone, or Bluetooth  device, and use it again later, simply touch Disconnect  on the Bluetooth settings screen. If you no longer want  to use your device with Model 3, touch Forget This  Device. Once you forget a device, you must pair it again  if you want to use it with Model 3 (see Pairing a  Bluetooth Device on page 52).

NOTE: Your phone automatically disconnects when you  leave Model 3.

NOTE: Unpairing the phone has no effect on using the  phone as a key. To forget an authenticated phone, see  Managing Keys on page 20.

Connecting to a Paired Device

Model 3 automatically connects to a phone that you  designated as Priority Device on the Bluetooth settings  screen. If you have not set a phone as a priority, Model 3  connects to the last phone to which is was connected,  provided it is within operating range and has Bluetooth  turned on. If the last phone is not within range, it  attempts to connect with the next phone that it has  been paired with.

To connect to a different phone, touch the Bluetooth  icon at the top of the Controls screen. The Bluetooth  settings screen displays a list of paired phones. Choose  the phone you want to connect to, then touch Connect.  If the phone you want to connect to is not listed, you  must pair the phone. See Pairing a Bluetooth Device on  page 52.

When connected, the Bluetooth settings screen displays  the Bluetooth symbol next to the phone's name to show  that Model 3 is connected to the phone.

Troubleshooting Bluetooth

Your vehicle uses Bluetooth and BLE (Bluetooth Low  Energy) to seamlessly connect your smartphone to  Model 3. Due to several potential factors, Bluetooth or  BLE may sometimes disconnect or experience issues in  the pairing process. Connecting to Bluetooth allows your  vehicle to use phone functions such as audio, phone  calls, calendars, text messages, etc.

BLE is used for passive functions like phone key.

NOTE: Do not unpair your vehicle to your phone or  remove it as phone key without a working key card  nearby.

Try the following to troubleshoot Bluetooth, starting  with your smartphone.

Bluetooth

Smartphone Troubleshooting

Bluetooth may not connect due to settings and updates  on your smartphone:

• Enable Bluetooth on your phone. If already enabled,  disable and re-enable Bluetooth again.

• Ensure Airplane Mode is turned off.

• Charge your phone; if your phone battery is too low,  it may not support Bluetooth functions.

• Pair your device properly. If already paired, try  unpairing and re-pairing again.

• Update your phone to the latest software provided  by the manufacturer.

• Check that your vehicle's sound system is selected as  the audio output source.

• Ensure your phone's settings allow for Bluetooth (ex:  data is turned on or you are connected to Wi-Fi).

• Turn your phone off and on again.

• Ensure location permissions set to “Always On” for  the mobile app.

Tesla Mobile App Troubleshooting Check the Tesla mobile app:

• Confirm the Tesla mobile app is up to date on  software.

• Verify you're logged into the Tesla mobile app while  using your phone key.

• Ensure the Tesla app is running in the background.

• Double check that you have completely set up your  profile in the mobile app and properly configured your settings.

Vehicle Troubleshooting

Your vehicle's settings may affect its ability to pair with  your smartphone:

• Charge Model 3: If the vehicle Battery is too low, you  may lose Bluetooth function.

• Update vehicle software and make sure it is always  up to date. Check for new software updates available  by navigating to Controls > Software

• Reboot your vehicle (see Restarting the Touchscreen  on page 8 for more information).

If Bluetooth still does not work, unpair from your vehicle  AND smartphone. Then try re-pairing both again.

Connectivity 53

Bluetooth

For BLE phone key issues, when in the vehicle, navigate

to Controls > Locks and remove your phone as "Phone

as Key". Then set it back up again. But only do this while

you are in the vehicle and have a reliable back up key

available (such as a key card).

54 MODEL 3 Owner's Manual

Phone, Calendar, and Web Conferencing NOTE: You can also make a phone call by touching a pin

Using the Phone App

When your phone is connected to Model 3

using Bluetooth (see Bluetooth on page

52), and you have allowed access to

information on your phone (see Importing

Contacts and Recent Calls on page 52),

you can use the phone app to display and

make a hands-free call to anyone listed on

your phone.

• Calls: Displays recent calls in chronological order with  the most recent call listed first.

• Messages: Displays message in chronological order  with the most recent message listed first. You can  view, send, and receive text messages. Instead of

typing a text message, touch the microphone button  on the right side of the steering wheel to enter text  using your voice.

WARNING: To minimize distraction and ensure  the safety of occupants as well as other road  users, do not view or send text messages when  the vehicle is in motion. Pay attention to road  and traffic conditions at all times when driving.

• Contacts: Contacts are listed in alphabetical order  and can be sorted by first name or last name. You can  also choose a letter on the right side of the list to  quickly scroll to the names that begin with the  selected character. When you touch a name on your  contacts list, the contact's available number(s)  displays on the right pane, along with other available  information (such as address). Touch the contact's  number to make a call.

• Favorites: Displays the contacts from your phone that  you have identified as Favorites.

• Calendar: Displays calendar entries from your phone  (see Calendar on page 55). If an entry includes a  phone number or an address, you can make a phone  call, or navigate to a destination, by touching the  corresponding information in the calendar entry.

Making a Phone Call

You can make a phone call by:

• Speaking a voice command (see Voice Commands on  page 16). Voice commands are a convenient, hands free way to call or text your contacts.

• Touching a phone number shown in a list in the  phone app - Contacts, Calls, or Calendar.

• Using the Model 3 on-screen dialer in the Phone app.

NOTE: If it is safe and legal to do so, you can also initiate  a call by dialing the number or selecting the contact  directly from your phone.

on the map and choosing the phone number (if  available) on the popup screen.

Receiving a Phone Call

When your phone receives an incoming call, the  touchscreen displays the caller’s number or name (if the  caller is in your phone’s contact list and Model 3 has  access to your contacts).

Touch one of the options on the touchscreen to Answer  or Ignore the call. Depending on the phone you are  using and what speakers you used for your most recent  call, your phone may prompt you to choose which  speakers you want to use for the incoming call.

WARNING: Stay focused on the road at all times  while driving. Using or programming a phone while  driving, even with Bluetooth enabled, can result in  serious injury or death.

WARNING: Follow all applicable laws regarding the  use of phones while driving, including, but not  limited to, laws that prohibit texting and require  hands-free operation at all times.

In Call Options

When a call is in progress, the call menu displays on the  touchscreen. Roll the right scroll button to choose an  option. To adjust the call volume, roll the left scroll  button during a call.

Calendar

The calendar displays scheduled events

from your phone’s (iPhone® or Android™)

calendar for the current and next day. The

calendar is conveniently integrated with

the phone app so you can dial into your

meeting from a Calendar entry. It is also

integrated with the navigation system so

you can navigate to the event's location.

1. Ensure your phone is paired to Model 3.

2. Ensure your are logged into the Tesla mobile app.

3. In your Tesla mobile app, touch Profile > Settings >  Calendar Sync.

NOTE: To ensure you have access to all of the  calendar's features, it is recommended that you use  the most recent version of the mobile app.

4. On your phone, go to Settings and allow access/give  permission to share your calendar with the Tesla  mobile app. The mobile app can then periodically  (and automatically) send calendar data from your  phone to Model 3.

Connectivity 55

Phone, Calendar, and Web Conferencing

If a calendar event includes an address, a navigation

arrow displays to indicate that you can touch the

address to navigate to the event's location.

If an event has a uniquely specified address and takes

place within two hours of you entering your vehicle and

preparing to drive, Model 3 automatically routes you to

the event's address (see Automatic Navigation on page

151).

Touch an event's information icon to display all notes

associated with the event. If the notes include one or

more phone numbers, the information icon shows a

phone icon and the calendar displays the first phone

number found. Touch to initiate a phone call. You can

also initiate a phone call by touching any number in an

event's notes popup screen (this is especially useful for

conference calls). If notes include a web link, you can

touch the link to open it in the Web browser.

Zoom

Seamlessly take meetings and calls

through your vehicle's touchscreen. To set

up, touch the Zoom app and sign in or

enter the meeting ID. You can even access

meetings shown on your calendar or in text

messages by touching the Zoom link. Your

vehicle's cabin camera can be used in calls

over Zoom only when Model 3 is Parked.

When the vehicle is shifted out of Park in

the middle of a Zoom call, the cabin

camera turns off and you switch to audio

only. Use the touchscreen to turn on/off

the video, mute/unmute yourself, and

customize various preferences for your

meeting.

WARNING: Do not to use the video function when

the vehicle is “temporarily parked” on a public

road (such as when the vehicle is parked along the

curb or in a spot that is not a designated parking

spot)..

WARNING: Stay focused on your surroundings and

follow all applicable laws while driving, including,

but not limited to, laws that require hands-free

operation at all times.

56 MODEL 3 Owner's Manual

HomeLink Universal Transceiver

If your vehicle is equipped with the

HomeLink® Universal Transceiver, you can

operate up to three Radio Frequency (RF)

devices, including garage doors, gates,

lights, and security systems.

NOTE: Depending on date of manufacture, market  region, and options selected at time of purchase, some  vehicles are not equipped with a HomeLink Universal  Transceiver.

WARNING: Do not use the HomeLink Universal  Transceiver with a device that does not have safety  stop and reverse features. Using a device without  these safety features increases the risk of injury or  death.

Supported Modes

HomeLink supports three different transmit modes,  which is how your vehicle and the RF device  communicate. Selecting a transmit mode is determined  by your RF device's compatibility:

• Standard Mode: Use Standard Mode if your RF device  is equipped with a remote control that must be used  to operate the device (for example, a remote controlled garage door). This mode is the most  commonly used transmit mode for HomeLink  devices.

• D-Mode or UR-Mode: Use D-Mode or UR-Mode if the  RF device does not have a remote control, and the  receiver has a "Learn" button (may also be called  "Program" or "Smart"). D-Mode and UR-Mode  function similarly in that Model 3 communicates  directly with the device's receiver as opposed to the  remote control.

NOTE: D-Mode is used primarily in North America  whereas UR-Mode is popular in Europe, the Middle  East, and Asia. To determine the mode your device is  compatible with, contact HomeLink

(www.homelink.com or call 1-800-355-3515).

Each of your devices can be set to a different mode. For  example, your garage door can be set to Standard  Mode, your front gate can be set to D-Mode, etc. To  change a transmit mode, touch the HomeLink icon at  the top of the Controls screen and select the device you  want to change. Then, select Program and choose the  desired mode for your device. Confirm by touching Set  Mode and follow the onscreen instructions.

For older vehicles, changing the mode for one device  changes the mode for all devices, so be careful when  changing transmit modes. Devices not compatible with  your selected mode may not work. Touch the HomeLink  icon at the top of the touchscreen, then touch Change  Transmit Mode.

Smart Garage

NOTE: Check the product information for your  HomeLink device to determine which mode is  compatible with your device.

Programming HomeLink

To program HomeLink®:

1. Park Model 3 so that the front bumper is in front of  the device you want to program.

CAUTION: Your device might open or close  during programming. Therefore, before

programming, make sure that the device is clear  of any people or objects.

2. Check that the device's remote control has a healthy  battery. Tesla recommends replacing the battery in  the device's remote control before Programming  HomeLink.

3. Touch the HomeLink icon at the top of the Controls  screen.

4. Touch Create HomeLink.

5. On the HomeLink screen, enter a name for the device,  then touch Enter or Add New HomeLink.

6. Choose the mode you wish to use (Standard, D Mode, or UR-Mode), then touch Set Mode.

7. Touch Start and follow the onscreen instructions.

NOTE: If you see a screen called “Train the receiver”  while programming the device, remember that this is  a time-sensitive step. After pressing the Learn/ Program/Smart button on the device's remote  control, you have only approximately 30 seconds to  return to your vehicle and press Continue, and then  press the trained HomeLink device name twice.  Consider having an assistant to ensure you can  complete this step within 30 seconds.

8. Once your device is programmed, touch Save to  complete the programming.

9. Ensure HomeLink works as expected. In some cases,  you may need to repeat the programming process  multiple times before succeeding.

Once programmed, you can operate the device by  touching its corresponding HomeLink icon on the  touchscreen. HomeLink remembers the location of your  programmed devices. When you approach a known  location, the HomeLink control on the touchscreen  automatically appears. When you drive away, it  disappears.

NOTE: The HomeLink icon displays at the top of the  touchscreen when Model 3 detects a programmed  HomeLink device within range, and the touchscreen is  not already displaying the HomeLink screen or popup,

NOTE: For additional assistance or compatibility  questions, contact HomeLink (www.homelink.com or call  1-800-355-3515).

Connectivity 57

Smart Garage

Auto Opening and Closing

To operate a HomeLink device without using the  touchscreen, you can automate the device to open as  you approach, and close as you drive away:

1. Touch the HomeLink icon at the top of the Controls  screen, touch HomeLink Settings, then choose the  device you want to automate.

2. Adjust the device's HomeLink settings as needed: ◦ Select the Auto-open when arriving checkbox if  you want the device to open as you approach.

◦ Touch the arrows to specify the distance you want  Model 3 to be from the device before it opens.

◦ Select the Auto-close when leaving checkbox if  you want the device to close as you drive away.

◦ Select the Auto-fold mirrors checkbox if you want  mirrors to fold when you arrive at the HomeLink  location. This is useful for narrow garages.

◦ Select the Chime for Auto-open and Auto-close  checkbox if you want Model 3 to sound a chime  when a signal has been sent to open or close the  device.

As you approach (or drive away from) a device that is  set to operate automatically, the HomeLink status icon  displays a count-down message to let you know when  the device automatically opens or closes. In situations

where you don't want the device to automatically open  or close, touch Skip Auto-Open or Skip Auto-Close at  any time during the count-down message.

NOTE: Do not rely on HomeLink to ensure the device  fully closes.

Resetting the Location of the  HomeLink Device

If you experience situations in which you sometimes  drive up to your HomeLink device and it doesn't open,  or the touchscreen does not display a notification as you  approach a programmed device, you may need to reset  the device's location. To do so, park as close as possible  to the HomeLink device (garage door, gate, etc.) and  display the HomeLink settings page by touching the  HomeLink icon at the top of the Controls screen. Touch  the name of the device you want to reset, then touch  Reset Location.

Deleting a Device

To delete a HomeLink device, touch the HomeLink icon  at the top of the Controls screen, then touch HomeLink  Settings. Touch the name of the device you want to  delete, then touch Delete.

NOTE: You can also perform a factory reset to erase  your HomeLink settings, along with all other personal  data (saved addresses, music favorites, imported  contacts, etc.). See Erasing Personal Data on page 10.

NOTE: For security reasons, delete your HomeLink  devices if you sell your Model 3.

Troubleshooting HomeLink

Standard Mode

In Standard Mode, Model 3 records the RF signal from  your HomeLink device's remote control. The  touchscreen instructs you to stand in front of the  vehicle, point the device's remote control at the front  bumper, and press and hold the button until the  headlights flash. When the headlights flash, Model 3 has  learned the remote control and you can touch Continue  on the touchscreen. If the headlights do not flash:

• Check the batteries in the remote control. It is a good  idea to replace the batteries before you start  programming.

• Ensure you are standing in front of Model 3 with the  device's remote control positioned within five cm of  the Tesla emblem.

• Press and hold the button on your device's remote  control until the headlights flash. In some cases you  must hold the button on the remote control for up to  three minutes.

NOTE: Some HomeLink remote controls require  multiple short presses (approximately one second  each press) instead of one long duration press. If you  are unsuccessful after multiple attempts of using long  presses, try repeated presses of one second each.

D-Mode and UR-Mode

In D-Mode and UR-Mode, the device’s receiver learns  Model 3. The touchscreen instructs you to press the  "Learn" button (may also be called "Program" or  "Smart") on the device's receiver. If this does not work,  refer to the following guidelines:

• Park Model 3 with its bumper as close as possible to  the garage door, gate, etc. that you are trying to  program.

• Make sure you are pressing the receiver's Learn/ Program/Smart button. For instructions on how to  put the receiver into learning mode, refer to the  product details provided with your RF device that  you are trying to program.

• If you see a screen called "Train the receiver" while  programming the device, remember that this is a  time-sensitive step. After pressing the Learn/ Program/Smart button on the device's remote  control or receiver, you only have approximately 30

58 MODEL 3 Owner's Manual

Smart Garage

seconds to return to your vehicle, press Continue,

then press the trained HomeLink device name twice.

Consider having someone assist you with this step.

• Most devices stay in learning mode for only three to

five minutes. Immediately after pressing the device's

Learn/Program/Smart button, follow the instructions

displayed on the vehicle's touchscreen.

For additional assistance or compatibility questions,

contact HomeLink (www.homelink.com or call

1-800-355-3515).

Connectivity 59

Starting and Powering Off Starting

When you open a door to enter Model 3, the

touchscreen powers on and you can operate all controls.  To shift Model 3, press the brake pedal and swipe up for  Drive or down for Reverse on the touchscreen's drive  mode strip (see Shifting on page 68).

If Auto Shift out of Park is enabled, Model 3

automatically selects Drive or Reverse based on the  detected surroundings . Pressing the brake pedal shifts  the vehicle into the selected drive mode displayed on  the touchscreen's drive mode strip (provided the driver's  door is closed and the driver's seat belt is buckled), and  pressing the accelerator moves the vehicle in that  direction.

NOTE: To turn Auto Shift out of Park on or off, touch  Controls > Pedals & Steering > Auto Shift out of Park.

Before accelerating when Auto Shift out of Park is  enabled, check the touchscreen to make sure that Model

3 has shifted into the drive mode you want (Drive or  Reverse). If the selection is not correct, or if Auto Shift  out of Park is not enabled, swipe up for Drive or down  for Reverse on the touchscreen's drive mode strip to  choose a new drive mode. See Shifting on page 68.

NOTE: If PIN to Drive is enabled (see PIN to Drive on  page 132), you must enter a valid PIN on the  touchscreen before you can drive Model 3.

Everything you need to know when driving Model 3  displays on the touchscreen.

Drive Disabled - Requires Authentication

If Model 3 does not detect a key when you press the  brake (a phone key is not detected or two minutes have  passed since you used the key card), the touchscreen  displays a message telling you that driving requires  authentication.

If you see this message, place the key card in either  phone dock where the RFID transmitter can read it. The  two-minute authentication period restarts and you can  start Model 3 by pressing the brake pedal.

A number of factors can affect whether Model 3 can  detect a phone key (for example, the device's battery is  low or dead and is no longer able to communicate using  Bluetooth).

Always keep your phone key or a key card with you.  After driving, your key is needed to restart Model 3. And  when you leave the vehicle, you must bring your key  with you to lock Model 3, either manually or  automatically.

Powering Off

When you finish driving and shift into Park, simply exit  the vehicle. When you leave Model 3 with your phone  key, it powers off automatically, turning off the  touchscreen.

Model 3 also powers off automatically after being in  Park for 30 minutes, even if you are sitting in the driver’s  seat.

Although usually not needed, you can power off Model 3  while sitting in the driver’s seat, provided the vehicle is  not moving. Touch Controls > Safety > Power Off. Model  3 automatically powers back on after a short period if  you press the brake pedal or touch the touchscreen.

NOTE: Model 3 automatically shifts into Park whenever it  determines that you are exiting the vehicle (for example,  the driver's seat belt is unbuckled and the vehicle is  almost at a standstill). If you shift into Neutral, your  vehicle shifts into Park when you open the door to exit.  To keep your vehicle in Neutral, you will need to activate  Transport Mode (see Instructions for Transporters on  page 203).

60 MODEL 3 Owner's Manual

Power Cycling the Vehicle

Starting and Powering Off

You can power cycle Model 3 if it demonstrates unusual

behavior or displays a nondescript alert.

NOTE: If the touchscreen is unresponsive or

demonstrates unusual behavior, reboot it before you

power cycle the vehicle (see Restarting the Touchscreen

on page 8).

1. Shift into Park.

2. On the touchscreen, touch Controls > Safety > Power

Off.

3. Wait for at least two minutes without interacting with

the vehicle. Do not open the doors, touch the brake

pedal, touch the touchscreen, etc.

4. After two minutes, press the brake pedal or open the

door to wake the vehicle.

Driving 61

Steering Wheel

Adjusting the Steering Wheel

Position

To adjust the steering wheel, touch Controls and touch

the Steering icon.

Use the left scroll button on the steering wheel to move

the steering wheel to the desired position:

• To adjust the height/tilt angle of the steering wheel,

roll the left scroll button up or down.

• To move the steering wheel closer to you, or further

away from you, press the left scroll button to the left

or right.

WARNING: Do not make steering wheel

adjustments while driving.

Adjusting Steering Effort

You can adjust the feel and sensitivity of the steering

system to suit your personal preference:

1. On the touchscreen, touch Controls > Pedals &

Steering > Steering Mode.

2. Choose a steering option:

◦ Comfort - Reduces the effort required to turn the

steering wheel. In town, Model 3 feels easier to

drive and park.

◦ Standard - Tesla believes that this setting offers

the best handling and response in most

conditions.

◦ Sport - Increases the effort required to turn the

steering wheel. When driving at higher speeds,

Model 3 feels more responsive.

62 MODEL 3 Owner's Manual

Steering Wheel

Steering Wheel Overview

Your Model 3 features stalkless driving in which all controls you need when driving are accessible on the steering  wheel.

1. Left turn signal (see Turn Signals on page 72)

2. Right turn signal (see Turn Signals on page 72)

3. High beam headlights (see High Beam Headlights on page 71)

4. Cruise control light (non-functional)

5. Rear-facing camera button

6. Wipers (see Wipers and Washers on page 74)

7. Voice commands (see Voice Commands on page 16)

8. Right scroll wheel (see Right Scroll Wheel (Autopilot) on page 64)

9. Horn (see Horn on page 65)

10. Left scroll wheel (see Left Scroll Wheel (Multifunction) on page 64 )

Driving 63

Steering Wheel

Scroll Wheels

The scroll wheel on the left side of the steering wheel controls functions such as mirror adjustment, volume control,  and wiper speed. The right scroll wheel controls Autopilot features, such as Autosteer initiation and max speed limit.

Left Scroll Wheel (Multifunction)

| Mode | Action | Function | More Information |
| --- | --- | --- | --- |
| Normal | Press | Play/pause media | Media on page 155 |
|  | Tilt left/right | Next/previous track |  |
|  | Scroll up/down | Adjust volume |  |
| Wipers* | Press | Confirm speed | Wipers and Washers on page 74 |
|  | Tilt left/right | Select wiper speed |  |
| Multifunction | Long press | Activate/set function | Left Scroll Wheel Customization on page 64 |
|  | Tilt left/right | Navigate list |  |
|  | Scroll | Select function |  |
| Phone call | Tilt left/right | Answer/decline a phone call | Using the Phone App on page 55 |
|  | In a call, tilt left | Mute/unmute |  |
|  | In a call, tilt right | End a call |  |

* First press the wiper button on steering wheel to activate.

Left Scroll Wheel Customization

Choose from a list to create quick access to a function by touching Controls > Display > Scroll Wheel Function.  Selecting a function sets the default action when you long press the left scroll button, unless you select Ask each  time.

Right Scroll Wheel (Autopilot)

| Mode | Action | Function | More Information |
| --- | --- | --- | --- |
| Single Click* | Press | Activate Autosteer | Autopilot Settings on page 98 |
|  | Tilt left/right | Adjust follow distance |  |
|  | Scroll up/down | Adjust max speed |  |
| Double Click* | Press | Activate Traffic-Aware Cruise Control |  |
|  | Double press | Activate Autosteer |  |
|  | Tilt left/right | Adjust follow distance |  |
|  | Scroll | Adjust max speed |  |

* Choose how you want Autopilot features to be enabled by touching Controls > Autopilot > Autopilot Activation. Both Scroll Wheels

| Mode | Action | Function | More Information |
| --- | --- | --- | --- |
| Normal | Press both briefly | Activate secondary drive mode selector (P, R, N, D) | Shift Using the Overhead Console on page 68 |
|  | Press both and hold | Restart the touchscreen | Restarting the Touchscreen on page 8 |

64 MODEL 3 Owner's Manual

Steering Wheel

Heated Steering Wheel

To warm up the steering wheel, touch the fan icon on

the touchscreen to display climate controls (see

Adjusting Climate Control Settings on page 139), then

touch the steering wheel icon. When on, radiant heat

keeps the steering wheel at a comfortable temperature.

Horn

To sound the horn, press and hold the center pad on the

steering wheel.

Driving 65

Mirrors

Adjusting Exterior Mirrors

Adjust the exterior mirrors by touching Controls >  Mirrors. Press the left scroll button on the steering wheel  to choose whether you are adjusting the Left or Right  mirror. Then use the left scroll button as follows to adjust  the selected mirror to its desired position:

• To move the mirror up or down, roll the left scroll  button up or down.

• To move the mirror inward or outward, press the left  scroll button to the left or right.

Both exterior mirrors can tilt downward when the  vehicle is shifted into Reverse. When you shift back into  another drive mode, the mirrors return to their normal  upward position. To turn this feature on or off, touch  Controls > Mirrors > Mirror Auto Tilt.

NOTE: With a future software update, the Save button  will retain the mirror adjustments but it does not  function currently.

To reduce glare when driving at night, the rear view  mirror and exterior side mirrors dim automatically. To  enable or disable this feature, touch Controls > Mirrors >  Mirror Auto Dim.

Availability of this Mirror Auto Dim depends on market  region and date of manufacture.

NOTE: Both exterior mirrors have heaters that turn on  and off with the rear window defroster.

Folding Mirrors

To manually fold and unfold exterior mirrors (for  example, parking in a narrow garage, tight space, etc.),  touch Controls > Fold/Unfold Mirrors.

When you manually fold the mirrors, they remain folded  until your driving speed reaches 50 km/h (or until you  manually unfold them by touching Controls > Unfold  Mirrors).

NOTE: You cannot fold a mirror when driving over  50 km/h.

To set the mirrors to fold automatically whenever you  exit and lock Model 3 touch Controls > Mirrors > Mirror  Auto Fold. The mirrors unfold automatically when you  unlock Model 3.

You can also set mirrors to fold automatically whenever  you arrive at a specific location, which saves you from  having to manually fold them each time you arrive at a  frequented place. To set this up, stop at the location you  want to save (or drive at less than 5 km/h), and fold the  mirrors. Save Location appears briefly below the Fold  Mirrors control. Touch again if you no longer want  mirrors to automatically fold at the location, or disable  this option in Controls.

NOTE: When you leave the saved location, mirrors can  unfold when your driving speed is over 5 km/h, or until  you touch Controls > Unfold Mirrors.

NOTE: Mirrors can automatically fold if you return to a  saved location and are driving below 50 km/h.

NOTE: You can override the automatic folding/unfolding  of mirrors at any time (for example, Model 3 has no  power) by pushing the mirror assembly away from you  to unfold, or pulling it toward you to fold.

NOTE: If you expect ice to accumulate when Model 3 is  parked, turn off Mirror Auto Fold. Accumulation of ice  can prevent exterior side mirrors from folding or  unfolding. See Cold Weather Best Practices on page 145  for information on how to ensure your mirrors function  properly in cold weather.

You can integrate auto-folding mirrors with HomeLink  (see Smart Garage on page 57). To enable, go to  HomeLink > Auto-Fold Mirrors when Nearby.

Rear View Mirror

Adjust the rear view mirror manually. When in Drive or  Neutral, the rear view mirror automatically dims in low  lighting conditions based on the time of day (for  example, when driving at night).

Vanity Mirrors

To expose and illuminate the vanity mirror, fold the sun  visor downwards, then use the tab to lower the mirror  cover. After closing the mirror cover, the light turns off.

66 MODEL 3 Owner's Manual

Mirrors

Driving 67

Shifting

Shift Using the Touchscreen

When you press the brake pedal when parked, the drive  mode strip displays on one side of the touchscreen. Use  the drive mode strip to shift Model 3: swipe up for Drive,  swipe down for Reverse or touch the P for Park or N for

Neutral. Swipe from the edge of the touchscreen  towards the passenger to bring up the drive mode strip.

NOTE: To shift from Drive into Reverse or vice versa, the  driving speed must be less than 8 km/h.

The touchscreen's drive mode strip displays Park and  Neutral at all times. To shift into Park when driving  below 8 km/h, touch the button on the drive mode strip  while pressing the brake pedal. In emergency situations  when driving above 8 km/h, press and hold the Park  button to slowly bring the vehicle to a stop.

The touchscreen is the preferred method to manually  shift. However, in the unlikely situation in which the  touchscreen is unavailable and therefore can't be used,  the drive mode selector on the overhead console  automatically activates and must be used to shift (see  Shift Using the Overhead Console on page 68).

If you try to shift when it is prohibited by the current  driving speed, the touchscreen displays an alert, a chime  sounds, and the drive mode does not change.

Auto Shift out of Park

Auto Shift out of Park is a BETA feature and is disabled  by default. When disabled, use the touchscreen or the  overhead console to manually shift. To enable Auto Shift  out of Park, touch Controls > Pedals & Steering > Auto  Shift out of Park.

When Auto Shift out of Park is enabled, Model 3 is  designed to automatically select Drive or Reverse. The  touchscreen displays the selected drive mode when the  driver's door is closed and seat belt is buckled.

To override the selection, press the brake pedal and use  the drive mode strip on touchscreen to shift into your  desired drive mode (Drive, Reverse, Park; see Shift Using  the Touchscreen on page 68).

Confirm the drive mode selection and follow the  instructions on the touchscreen before you press the  accelerator.

WARNING: As always, be aware of your vehicle  and surroundings before driving. Never rely on  Model 3 to automatically select a suitable drive

mode without confirming the selection before you  start to drive.

If Auto Shift out of Park is unavailable, the touchscreen  displays a message.

Model 3 automatically selects a drive mode when:

• Auto Shift out of Park is enabled.

• Model 3 is in Park.

• The driver's seat belt is fastened.

• The brake pedal is pressed.

• All doors and trunks are closed.

• The drive mode selector on the overhead console is  not activated (see Shift Using the Overhead Console  on page 68).

NOTE: Model 3 does not automatically select drive  modes in Valet Mode.

Shift Using the Overhead Console

In addition to manually shifting on the touchscreen, you  can shift by pressing P, R, N or D located on the  overhead console. In most situations, these buttons are  not available until you press one of the buttons to  activate it. When active, the LEDs associated with each  button illuminate and when you select P, R, N or D, the  associated LED illuminates amber.

In situations where the touchscreen is unavailable (for  example, experiencing a technical issue), or Model 3 is in  Valet or Transport Mode, the drive mode selector on the  overhead console activates automatically and there is no  need to touch it.

68 MODEL 3 Owner's Manual

NOTE: You can also activate the drive mode selector on  the overhead console by simultaneously and briefly pressing both scroll buttons on the steering wheel.  However, if you press and hold both buttons  simultaneously, the drive mode selector activates and  the touchscreen restarts (see Restarting the  Touchscreen on page 8).

NOTE: The front trunk must be closed to shift using the  overhead console.

1. Park

2. Reverse

3. Neutral

4. Drive

NOTE: When the touchscreen is available for shifting  and you have manually activated the drive mode  selector on the overhead console, the overhead console  automatically deactivates if you don't shift within 10  seconds.

Park

To manually shift into Park, press the brake pedal and  touch the Park button on the touchscreen's drive mode  strip. If the touchscreen is unavailable, press Park on the  drive mode selector located on the overhead console.

Model 3 automatically shifts into Park to prevent roll away while driving in low speeds. This happens  whenever you connect a charge cable, unbuckle your  seat belt, or open the door while in Drive or Neutral.  Ensure the charge cable is removed, buckle your seat  belt, and close the door before shifting out of Park.

Attempting to engage the parking brake above 8 km/h  will result in emergency braking (see Braking and  Stopping on page 75).

NOTE: In emergency situations, if the brakes are not  functioning properly, you can press and hold the Park  button on the touchscreen's drive mode strip to bring  Model 3 to a stop. Do not use this method to stop the  vehicle unless absolutely necessary.

Shifting

NOTE: You must always press the brake pedal to shift  out of Park.

CAUTION: Model 3 will not shift out of Park if a  charge cable is plugged in, or if the charge port is  unable to determine whether a charging cable is  plugged in. In situations when Model 3 does not  shift out of Park, check the touchscreen for  instructions on how to proceed.

WARNING: It is the driver's responsibility to always  ensure the vehicle is in Park before exiting. Never  rely on Model 3 to automatically shift into Park for  you.

Drive

To manually shift into Drive, swipe up on the drive mode  strip located on the touchscreen or, if the touchscreen is  unavailable, press D on the drive mode selector located

on the overhead console. You can shift into Drive when  Model 3 is stopped or moving less than 8 km/h in  Reverse.

Reverse

To manually shift into Reverse, swipe down on the drive  mode strip located on the touchscreen or, if the  touchscreen is unavailable, press R on the drive mode  selector located on the overhead console. You can shift  into Reverse when Model 3 is stopped or moving less  than 8 km/h in Drive. You can manually close the park  assist view on the touchscreen by touching the X in the  upper corner.

Neutral

Neutral allows Model 3 to roll freely when you are not  pressing the brake pedal. To shift into Neutral, do any of  the following:

• Swipe from the edge of the touchscreen towards the  passenger to bring up the drive mode strip and press  Neutral.

• Choose Neutral from the drive mode selector on the  overhead console.

NOTE: You must press the brake pedal to shift out of  Neutral if Model 3 is moving slower than approximately 8  km/h.

Model 3 automatically shifts into Park when you open  the driver's door to exit the vehicle unless the vehicle is  in certain modes such as Transport or Car Wash Mode  which allows the vehicle to stay in Neutral even when  you leave.

Driving 69

Shifting

Keeping Your Vehicle in Neutral

(Transport Mode)

To keep Model 3 in Neutral when you exit, allowing it to

roll freely (for example, pulling onto a flatbed truck), you

must activate Transport mode:

1. Shift into Park.

2. Press the brake pedal.

3. Touch Controls > Service > Towing. The touchscreen

displays a message reminding you how to properly

transport Model 3.

4. Press the Transport Mode. It turns blue to show that

Model 3 is now in Transport Mode. Model 3 is now

free-rolling and can slowly be rolled (no faster than

walking speed) for short distances or winched (for

example, onto a flatbed truck).

For more information on Transport Mode, see Activate

Transport Mode on page 203.

NOTE: In Transport mode, Model 3 does not shift into a

drive mode. You must first cancel Transport mode by

touching Transport Mode again. Transport mode also

cancels if you use the touchscreen or drive mode

selector on the overhead console to shift into Park or if

you manually apply the parking brake by touching

Controls > Safety > Parking Brake.

70 MODEL 3 Owner's Manual

Controlling Lights

Touch Controls > Lights to control the lights.

NOTE: You can also access an abbreviated lights menu  while driving by touching the High Beam button on the  steering wheel. A lights menu displays on the  touchscreen, providing quick access to headlight  controls.

In addition to the lights that you can control from the  touchscreen, Model 3 has convenience lights that turn  on and off automatically based on what you are doing.  For example, interior lights, marker lights, tail lights, and  puddle lights turn on when you unlock Model 3, when  you open a door, and when you shift into Park. They turn  off automatically after a minute or two or when you shift  or lock Model 3.

Exterior Lights

Exterior lights (headlights, tail lights, side marker lights,  parking lights, and license plate lights) are set to Auto  each time you start Model 3. If you change to a different setting, lights always revert to Auto on your next drive.

Touch one of these options to change and retain the  exterior light setting until adjusted again or the next  time you drive:

OFF Exterior lights turn off. When driving,  daytime running lights may remain on

based on regulations in various market

regions.

Parking lights, side marker lights, tail lights  and license plate lights turn on.

Low beam headlights, side marker lights,

parking lights, tail lights, and license plate

lights turn on.

CAUTION: Ensure the headlights and rear lights  are on during low visibility conditions (for example,  when it is dark, foggy, snowy, or the road is wet,  etc.). The rear tail lights are off while daytime  running lights are on. Failure to do so can cause  damage or serious injury.

Dome Lights

Turn the interior dome (map) lights on or off. If set to  Auto, all interior dome lights turn on when you unlock  Model 3, open a door upon exiting, or shift into Park.

Lights

You can also manually turn an individual dome light on  or off by pressing its lens. If you turn a dome light on, it  turns off when Model 3 powers off. If Model 3 was  already powered off when you manually turned the light  on, it turns off after 60 minutes.

Accent Lights

Enable or disable the accent lights that line the cabin of  your vehicle and customize the accent light color to your  preferences (if equipped). You can also turn the footwell  lights on or off.

Headlights after Exit

When you stop driving and park Model 3 in low lighting  conditions, exterior lights temporarily remain  illuminated. They automatically turn off after one minute  or when you lock Model 3 whichever comes first.

NOTE: If you lock Model 3 using the Tesla mobile app ,  the headlights immediately turn off. However, if the  vehicle locks because Walk-Away Door Lock is enabled  (see ), the headlights automatically turn off after one  minute.

To turn this feature on and off, touch Controls > Lights >  Headlights After Exit. When Headlights After Exit is off, the headlights turn off immediately when you engage  Park and open a door.

High Beam Headlights

Use the high beam headlight button on the left side of  the steering wheel to control the headlights:

• Press and quickly release to flash high beam  headlights.

• Press and hold to turn on high beam headlights - the  touchscreen displays a brief timer and you must hold  for the duration of the timer to latch the high beam  headlights to the on position. When headlights are  on, press the button a second time to turn them off.

Driving 71

Lights

Auto High Beam (if equipped)

The high beam headlights can automatically switch to  low beam when there is light detected in front of Model  3 (for example, from an oncoming vehicle). To turn this  feature on or off, touch Controls > Lights > Auto High  Beam. Your chosen setting is retained until you manually  change it.

NOTE: When you touch or press the high beam  headlight button on the left side of the steering wheel,  the touchscreen displays an abbreviated lights menu to  provide quick access to control headlights, fog lights (if  equipped), and the Auto High Beam setting. Use the  touchscreen to choose options from this menu.

NOTE: Auto High Beam is automatically enabled when  Autosteer is engaged. To switch to low beam headlights,  press the high beam headlight button on the steering  wheel. Auto High Beam is re-enabled every time  Autosteer is activated.

The following indicator lights are visible on the  touchscreen to show the status of the headlights:

Low beam headlights are on.

High beam headlights are on. Illuminates

when high beams are on but the Auto High  Beam setting is turned off or if the Auto

High Beam setting is turned on but is

temporarily unavailable.

High beams are currently turned on, and

Auto High Beam is ready to turn off the

high beams if light is detected in front of

Model 3.

High beams are temporarily turned off

because Auto High Beam is operating and

light is detected in front of Model 3. When

light is no longer detected, high beam

headlights automatically turn back on.

WARNING: Auto High Beam is a convenience only  and is subject to limitations. It is the driver's  responsibility to make sure that headlights are  always appropriately adjusted for weather

conditions and driving circumstances.

Headlight Adjustments

To adjust the angle of the headlights, touch Controls >  Service > Adjust Headlights, then follow the onscreen  instructions. You can choose which headlight you would  like to adjust by selecting it on the touchscreen.

NOTE: Headlights do not require adjustments when  temporarily driving into a region where the traffic direction is different (for example, driving in right-hand  traffic region, and then driving into a region with left hand traffic).

WARNING: Proceed with caution when adjusting  headlights. Tesla has carefully calibrated the  position of the headlights to be in an optimum  position for most driving scenarios. Tesla

recommends that you do not adjust headlights  unless you are familiar with how headlights should  be adjusted. Once adjusted, you will be unable to  automatically restore them to their originally  calibrated position. Contact Tesla for assistance  when adjusting headlights.

Turn Signals

To engage a turn signal, press the corresponding arrow  button on the left side of the steering wheel. A turn  signal cancels based on the angle of the steering wheel  (for example, you finish making a turn). You can also  cancel a turn signal by pressing the turn signal button a  second time.

If Controls > Lights > Automatic Turn Signals is set to  Auto Cancel, turn signals cancel automatically when  Model 3 detects completion of a maneuver such as a  merge, lane change, or a fork in the roadway. You can  override automatic cancellation at any time (for  example, you want the turn signal to remain on because  you are making more than one lane change). To override,  engage the turn signal by pressing and momentarily  holding the turn signal button (instead of just pressing).  Then, when the first maneuver is complete, the turn  signal remains on. If Automatic Turn Signals is set to Off,

72 MODEL 3 Owner's Manual

you must cancel the turn signal manually by pressing the  turn signal button after maneuvers such as a merge, lane  change, or fork in the roadway.

When a turn signal is operating, the

corresponding indicator lights up on the

touchscreen and you can hear a clicking

sound.

Hazards

To turn on the hazard warning flashers, press the button  on the drive mode selector located on the overhead  center. All turn signals flash. Press again to turn off.

NOTE: Hazard warning flashers operate even without a  key nearby.

Lights

Condensation in Head or Tail Lights

Due to weather changes, humidity levels, or recent  exposure to water (such as a car wash), condensation  may occasionally accumulate in your vehicle’s head or  tail lights. This is normal— as the weather gets warmer  and humidity decreases, condensation often disappears  on its own. If you notice water buildup within the  exterior lenses, or if the condensation affects the  visibility of the exterior lights, contact Tesla Service.

Driving 73

Wipers and Washers

Wipers and Washers

You can access wiper settings by touching the wiper  button on the steering wheel or touching Controls >  Wipers.

Press the wiper button on the steering wheel to wipe the  windshield.

Press and hold the wiper button to spray washer fluid onto the windshield. After releasing the button, the  wipers perform two additional wipes then, depending on  vehicle and environmental conditions, a third wipe a few  seconds later. You can also press and hold the wiper  button for a continuous spray of washer fluid—the wipers perform the wipes after you release.

Whenever you press the wiper button on the steering  wheel, the touchscreen displays the wiper menu,  allowing you to adjust wiper settings. Press the left scroll  button on the steering wheel left or right to choose your  desired setting.

1. Turn the wipers off.

2. Choose how you want the wipers to operate: ◦ Auto - Model 3 detects whether or not it is raining.  When liquid is detected on the windshield, Model  3 determines the optimal frequency of the wipers.  If Model 3 does not detect liquid on the

windshield, the wipers do not operate.

NOTE: Autosteer and traffic-aware cruise control  require wipers to be set to Auto. This setting is  automatic and you cannot change it.

NOTE: The Auto setting is currently in BETA. If  uncertain about using the Auto setting while in the  BETA phase, Tesla recommends operating the  wipers manually, as necessary.

◦ I - Intermittent, slow.

◦ II - Intermittent, fast.

◦ III - Continuous, slow.

◦ IIII - Continuous, fast.

NOTE: If you choose Auto and the ability to detect  liquid on the windshield becomes unavailable, the  wiper setting reverts to the most recently used  manual setting. If a manual setting cannot be  determined, the wipers turn off.

NOTE: You can also adjust the windshield wiper  speed and frequency using voice commands (see  Voice Commands on page 16).

CAUTION: To avoid the risk of damaging the  wipers, do not leave the wipers set to Auto  when washing Model 3.

Periodically check and clean the edge of the wiper  blades. If a blade is damaged, replace it immediately. For  details on checking and replacing wiper blades, see  Windshield Wiper Blades, Jets and Fluid on page 187.

CAUTION: To avoid damaging the hood, ensure  that the hood is fully closed before activating the  windshield wipers.

CAUTION: Remove ice from the windshield before  turning the wipers on. Ice has sharp edges that can  damage the rubber on the wiper blades.

CAUTION: In harsh climates, ensure that the wiper  blades are not frozen or adhered to the windshield.

74 MODEL 3 Owner's Manual

Braking Systems

WARNING: Properly functioning braking systems  are critical to ensure safety. If you experience a  problem with the brake pedal, brake calipers, or  any component of a Model 3 braking system,  contact Tesla immediately.

Model 3 has an anti-lock braking system (ABS) that  prevents the wheels from locking when you apply  maximum brake pressure. This improves steering control  during heavy braking in most road conditions.

During emergency braking conditions, the ABS  constantly monitors the speed of each wheel and varies  the brake pressure according to the grip available.

The alteration of brake pressure can be felt as a pulsing  sensation through the brake pedal. This demonstrates  that the ABS is operating and is not a cause for concern.  Keep firm and steady pressure on the brake pedal while  experiencing the pulsing.

The ABS indicator briefly flashes amber on  the touchscreen when you first start Model  3. If this indicator lights up at any other

time, an ABS fault has occurred and the

ABS is not operating. Contact Tesla. The

braking system remains fully operational

and is not affected by an ABS failure.

However, braking distances may increase.

Drive cautiously and avoid heavy braking.

If the touchscreen displays this red brake

indicator at any time other than briefly

when you first start Model 3, a brake

system fault is detected, or the level of the  brake fluid is low. Contact Tesla

immediately. Apply steady pressure and

keep the brakes firm to bring the vehicle to  a stop when safe to do so.

The touchscreen displays this amber brake  indicator if a brake booster fault is

detected. Apply steady pressure and keep

the brakes firm to stop the vehicle when

safety permits. Hydraulic Boost

Compensation will be active (see Hydraulic  Boost Compensation on page 76).

Emergency Braking

In an emergency, fully press the brake pedal and  maintain firm pressure, even on low traction surfaces.  The ABS varies the braking pressure to each wheel  according to the amount of traction available. This  prevents wheels from locking and ensures that you stop  as safely as possible.

Braking and Stopping

If an alternative method is needed to bring the vehicle to  a stop, press and hold the Park button on the  touchscreen's drive mode strip to apply the brakes and  remove drive torque while the button is held. Swipe from  the edge of the touchscreen towards the passenger, to  bring up the drive mode strip.

WARNING: Do not pump the brake pedal. Doing so  interrupts operation of the ABS and can increase  braking distance.

WARNING: Always maintain a safe distance from  the vehicle in front of you and be aware of

hazardous driving conditions. While the ABS can  improve stopping distance, it cannot overcome the  laws of physics. It also does not prevent the  danger of hydroplaning (where a layer of water  prevents direct contact between the tires and the  road).

CAUTION: Automatic Emergency Braking (see  Collision Avoidance Assist on page 127) may  intervene to automatically brake in situations  where a collision is considered imminent.

Automatic Emergency Braking is not designed to  prevent a collision. At best, it can minimize the  impact of a frontal collision by attempting to  reduce your driving speed. Depending on

Automatic Emergency Braking to avoid a collision  can result in serious injury or death.

CAUTION: In emergency situations, if the brakes  are not functioning properly, press and hold the  Park button on the overhead console or

touchscreen to bring Model 3 to a stop. Do not use  this method to stop the vehicle unless absolutely  necessary.

Dynamic Brake Lights (if equipped)

If you are driving over 50 km/h and brake forcefully (or  if Automatic Emergency Braking engages), the brake  lights flash quickly to warn other drivers that Model 3 is  rapidly slowing down. If Model 3 stops completely, the  hazard warning lights flash. Flashing continues until you  press the accelerator or manually press the hazard lights  button to turn them off (see Hazards on page 73).

NOTE: When towing a trailer (if applicable), the brake  lights on the trailer also operate as described above,  even when the trailer is not equipped with a separate  braking system.

WARNING: When towing a trailer (if applicable),  always increase your following distance. Sudden  braking may result in skidding, jack-knifing, and  loss of control.

Driving 75

Braking and Stopping Brake Disc Wiping

To ensure brakes remain responsive in cold and wet  weather, Model 3 is equipped with brake disc wiping.  When cold and wet weather is detected, this feature  repeatedly applies an imperceptible amount of brake  force to remove water from the surface of the brake  discs.

Hydraulic Fade Compensation

Model 3 is equipped with hydraulic fade compensation.  This assists in monitoring brake system pressure and  ABS activity for instances of reduced brake  performance. If reduced brake performance is detected  (for example, as a result of brake fade, or cold or wet  conditions), you may hear a sound, feel the brake pedal  pull away from your foot, and notice a strong increase in  braking. Brake as you normally would and continue to  press the brake pedal without releasing or pumping the  brakes.

CAUTION: In emergency situations, if the brakes  are not functioning properly, press and hold the  Park button on the overhead console or

touchscreen to bring Model 3 to a stop. Do not use  this method to stop the vehicle unless absolutely  necessary.

WARNING: Always maintain a safe driving distance  from the vehicle in front of you and exercise  caution when driving conditions are hazardous.  Brake disc wiping and hydraulic fade

compensation is not a substitute for adequately  applying the brakes.

Hydraulic Boost Compensation

Model 3 is equipped with a brake booster that activates  the brakes when the brake pedal is pressed. Hydraulic  boost compensation provides mechanical assistance if  the brake booster fails. If a brake booster failure is  detected, the brake pedal feels stiffer to press and you  may hear a sound when you press the brake pedal. To  stop Model 3, apply steady force to the brake pedal  without releasing or pumping. Drive cautiously and  maintain a safe distance from other road users—brake  pedal responsiveness and braking performance may be  degraded.

Regenerative Braking

Whenever Model 3 is moving and your foot is off the  accelerator, regenerative braking slows down the vehicle  and feeds any surplus power back to the Battery. By  anticipating your stops and reducing or removing  pressure from the accelerator pedal to slow down, you  can take advantage of regenerative braking to increase  driving range.

Vehicle deceleration due to regenerative braking may  vary depending on the current state of the Battery. For  example, regenerative braking may be limited if the  Battery is cold or is already fully charged.

The power meter (a thin horizontal line centered at the  top of the touchscreen's car status area) displays real time power usage:

1. The left side of the power meter represents power  generated from regenerative braking, or power that is  used to slow down the vehicle. Power being fed back  to the Battery displays in green whereas power used  by the regular braking system displays in gray.

2. The right side of the power meter shows power being  output by the Battery, such as that used to accelerate  the vehicle or to cool the cabin. When you press the  accelerator pedal, the power meter fills to the right  with black (or white if the display is dark).

NOTE: To ensure visibility when the display is dark,  power represented by the gray color displays as white.

NOTE: Installing winter tires with aggressive compound  and tread design may result in temporarily-reduced  regenerative braking power. However, your vehicle is  designed to continuously recalibrate itself, and after  changing tires it will increasingly restore regenerative  braking power after some straight-line accelerations. For  most drivers this occurs after a short period of normal  driving, but drivers who normally accelerate lightly may  need to use slightly harder accelerations while the  recalibration is in progress. Touch Service > Wheel &  Tire > Tires to select winter tires and quicken this  process.

NOTE: If regenerative braking is aggressively slowing  Model 3 (such as when your foot is completely off the  accelerator pedal at highway speeds), the brake lights  turn on to alert others that you are slowing down.

NOTE: Because Model 3 uses regenerative braking, the  brake pads are typically used less frequently than those  in traditional braking systems. To avoid the accumulation  of rust and corrosion, Tesla recommends frequently  pressing the brake pedal to apply the mechanical brakes  and dry the brake pads and rotors.

WARNING: In snowy or icy conditions, Model 3  may experience loss of traction during

regenerative braking.

WARNING: Never rely on your vehicle to

adequately decelerate or fully stop your vehicle.  Many factors can contribute to a longer stopping  distance, including downward slopes and reduced  or limited regenerative braking. Always be

prepared to use the brake pedal to adequately  decelerate or stop.

76 MODEL 3 Owner's Manual

WARNING: Press the brake pedal if Model 3 moves  when unsafe to do so. It is your responsibility to  stay alert and be in control of the vehicle at all  times. Failure to do so can result in serious  damage, injury, or death.

WARNING: Do not rely on regenerative braking to  keep you and your vehicle safe. Various factors  such as driving with a heavy vehicle load, on a  steep hill, or on wet or icy roads affect

deceleration rate and the distance at which Model  3 will come to a stop. Drive attentively and always  stay prepared to use the brake pedal to stop as  appropriate based on traffic and road conditions.

WARNING: Forward Collision Warning and  Automatic Emergency Braking do not operate  when driving at very low speeds (see Collision  Avoidance Assist on page 127). Do not rely on  these features to warn you, or to prevent or reduce  the impact of a collision.

Parking Brake

To engage the parking brake, touch Controls > Safety >  Parking Brake. Follow the onscreen instructions.

Use the touchscreen to manually release the parking  brake (which also shifts Model 3 into Neutral):

1. Touch Controls > Safety.

2. Press the brake pedal, then touch Parking Brake. If  Model 3 was previously in Park, it shifts into Neutral.

When you manually apply the parking

brake using the touchscreen, the red

parking brake indicator lights up on the

touchscreen.

If the parking brake experiences an

electrical issue, the amber parking brake

indicator lights up and a fault message

displays on the touchscreen.

NOTE: The parking brake operates on the rear wheels  only, and is independent of the pedal-operated brake  system.

CAUTION: In the unlikely event that Model 3 loses  electrical power, you cannot access the

touchscreen and are therefore unable to release  the parking brake without first jump starting (see  Jump Starting on page 210).

WARNING: In snowy or icy conditions the rear  wheels may not have sufficient traction to prevent  Model 3 from sliding down a slope, particularly if  not using winter tires. Avoid parking on hills in  snowy or icy conditions. You are always

responsible for parking safely.

Braking and Stopping

WARNING: Your Model 3 may display an alert if  the road is too steep to safely park on, or if the  parking brakes are not properly engaged. These  alerts are for guidance purposes only and are not a  substitute for the driver’s judgment of safe parking  conditions, including specific road or weather  conditions. Do not depend on these alerts to  determine whether or not it is safe to park at any  location. You are always responsible for parking  safely.

Brake Wear

Model 3 brake pads are equipped with wear indicators.  A wear indicator is a thin metal strip attached to the  brake pad that squeals as it rubs against the rotor when  the pad wears down. This squealing sound indicates that  the brake pads have reached the end of their service life  and require replacement. To replace the brake pads,  contact Tesla Service.

Brakes must be periodically inspected visually by  removing the tire and wheel. For detailed specifications and service limits for rotors and brake pads, see  Subsystems on page 197. Additionally, Tesla  recommends cleaning and lubricating the brake calipers  every year or 20,000 km if in an area where roads are  salted during winter months.

WARNING: Neglecting to replace worn brake pads  damages the braking system and can result in a  braking hazard.

Driving 77

Park Assist

How Park Assist Works

This feature may be temporarily limited or inactive until  it is enabled with a future software update for vehicles  manufactured as of approximately September 2023.

Model 3 is designed to detect the presence of objects.  When driving slowly (for example, when parking), the  vehicle alerts you if an object is detected in close  proximity of your Model 3. The vehicle alerts you when  objects are detected in front of Model 3 when you are in  Drive, and behind Model 3 when you are in Reverse.

WARNING: You may not be alerted if Model 3 rolls  freely in the opposite direction (for example, Park  Assist does not display an alert if Model 3 rolls  backwards down a hill while in Drive).

WARNING: Never depend on Park Assist to inform  you if an area you are approaching is free of  objects and/or people. Several external factors can  reduce the performance of Park Assist, causing  either no readings or false readings (see

Limitations and False Warnings on page 78).  Therefore, depending on Park Assist to determine  if Model 3 is approaching an obstruction can result  in damage to the vehicle and/or objects, and can  potentially cause serious injury. Always inspect the  area with your own eyes. When reversing, perform  shoulder checks and use all mirrors. Park assist  does not detect children, pedestrians, bicyclists,  animals, or objects that are moving, protruding,  located too far above or below the sensors (if  equipped) or cameras, or too close or too far from  the sensors or cameras. Park Assist is for guidance  purposes only and is not intended to replace your  own direct visual checks. It is not a substitute for  careful driving.

Visual and Audio Feedback

When you shift to Reverse, the Park Assist view displays  on the touchscreen, showing objects that are in close  proximity to the front and rear of Model 3. This view  closes when you shift into Drive unless an object is  detected close to the front of Model 3, in which case the  Park Assist view closes automatically when you start  driving faster than the speed at which Park Assist  operates. When reversing, visual feedback also displays  on the touchscreen (see Rear Facing Camera(s) on page  87). You can manually close the park assist view on the  touchscreen by touching the X.

When driving at low speeds with the Camera app  displayed on the touchscreen, you can switch to the  Park Assist view by touching the button located in the  upper left corner of the Camera app screen. This is  useful if you need assistance with parallel parking.

If chimes are turned on (see Controlling Audible  Feedback on page 78), an audible beep sounds as you  approach an object.

NOTE: If Park Assist is unable to provide feedback, the  touchscreen displays an alert message.

CAUTION: Keep sensors (if equipped) and  cameras clean from dirt, debris, snow, and ice.  Avoid using a high pressure power washer on the  sensors and cameras, and do not clean a sensor or  camera with a sharp or abrasive object that can  scratch or damage its surface.

CAUTION: Do not install accessories or stickers on  or near the sensors (if equipped) or cameras.

Controlling Audible Feedback

You can use Park Assist with or without audible  feedback. To turn chimes on or off, touch Controls >  Safety > Park Assist Chimes.

Limitations and False Warnings

Park Assist may not function correctly in these  situations:

• One or more of the sensors (if equipped) or cameras  is damaged, dirty, or obstructed (such as by mud, ice,  or snow, or by a vehicle bra, excessive paint, or  adhesive products such as wraps, stickers, rubber  coating, etc.).

• The object is located below approximately 20 cm  (such as a curb or low barrier).

CAUTION: Shorter objects that are detected  (such as curbs or low barriers) can move into a  blind spot. Model 3 cannot alert you about an  object while it is in a blind spot.

• Weather conditions (heavy rain, snow, or fog). • The object is thin (such as a sign post).

• Park Assist’s operating range has been exceeded.

• The object is sound-absorbing or soft (such as  powder snow).

• The object is sloped (such as a sloped embankment).

• Model 3 has been parked in, or being driven in,  extremely hot or cold temperatures.

• The sensors (if equipped) are affected by other  electrical equipment or devices that generate  ultrasonic waves.

• You are driving in a location where the sensors' (if  equipped) waves are deflected away from the vehicle  (such as driving next to a wall or pillar).

• The object is located too close to the bumper. • A bumper is misaligned or damaged.

• An object that is mounted to Model 3 is interfering  with and/or obstructing Park Assist (such as a bike  rack or bumper sticker).

78 MODEL 3 Owner's Manual